آيين نامهء سازه های فضاکار

پیوست 2

برنامه های فرميينی برای سازه های پیوست 1

برنامهء فرميينی برای تاشه های شبيه به شکل 1

(*) Formian scheme for configurations

similar to Fig 1:  Single layer grid with

binate pattern. See Reference 1. (*)

L1=25;    (*) Length in U1 direction (*)

L2=25;    (*) Length in U2 direction (*)

m=10;     (*) Frequency in U1 direction (*)

n=10;      (*) Frequency in U2 direction (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

Grid=bb(L1/m,L2/n)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 2

(*) Formian scheme for configurations

similar to Fig 2:  Single layer grid with

obnate pattern. See Reference 1. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=6;     (*) Frequency of crosses

in U1 direction (*)

n=6;      (*) Frequency of crosses

in U2 direction (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,2,2,2*n)|[0,0,0; 2,0,0]#

rinid(2,n,2*m,2)|[0,0,0; 0,2,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 3

(*) Formian scheme for configurations

similar to Fig 3:  Single layer grid with

binate pattern and additional diagonal

elements. See Reference 1. (*)

L1=25;    (*) Length in U1 direction (*)

L2=25;    (*) Length in U2 direction (*)

m=10;     (*) Frequency in U1 direction,

m must be even (*)

n=10;      (*) Frequency in U2 direction (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0]#

rinid(m/2,n,2,1)|lam(1,1)|[0,1,0; 1,0,0];

Grid=bb(L1/m,L2/n)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 4

(*) Formian scheme for configurations

similar to Fig 4:  Single layer grid with

binate pattern and additional diagonal

elements. See Reference 1. (*)

L1=25;    (*) Length in U1 direction (*)

L2=25;    (*) Length in U2 direction (*)

m=10;     (*) Frequency in U1 direction,

m must be even (*)

n=10;      (*) Frequency in U2 direction,

n must be even (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0]#

lam(2,n/2)|rinid(m/2,n/2,2,1)|lam(1,1)|

[0,1,0; 1,0,0];

Grid=bb(L1/m,L2/n)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 5

(*) Formian scheme for configurations

similar to Fig 5:  Single layer grid with

binate pattern and additional diagonal

elements. See Reference 1. (*)

L1=25;    (*) Length in U1 direction (*)

L2=25;    (*) Length in U2 direction (*)

m=10;     (*) Frequency in U1 direction,

m must be even (*)

n=10;      (*) Frequency in U2 direction,

n must be even (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0]#

lamid(m/2,n/2)|rinid(m/2,n/2,1,1)|

[0,1,0; 1,0,0];

Grid=bb(L1/m,L2/n)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 6

(*) Formian scheme for configurations

similar to Fig 6:  Single layer grid with

obnate pattern and additional diagonal

elements. See Reference 1. (*)

L1=25;   (*) Length in U1 direction (*)

L2=25;   (*) Length in U2 direction (*)

m=6;      (*) Frequency of crosses

in U1 direction (*)

n=6;       (*) Frequency of crosses

in U2 direction (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(2,n,2*m,2)|[0,0,0; 0,2,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 7

(*) Formian scheme for configurations

similar to Fig 7:  Single layer grid with

ternate pattern. See Reference 1. (*)

L1=25;   (*) Length in U1 direction (*)

L2=25;   (*) Length in U2 direction (*)

m=6;      (*) Frequency of crosses

in U1 direction (*)

n=6;       (*) Frequency of crosses

in U2 direction (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(2*m,2,1,2*n)|[0,0,0; 1,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0]#

rinid(m,2,2,2*n-1)|[1,0,0; 1,1,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 8

(*) Formian scheme for configurations

similar to Fig 8:  Single layer grid with

ternate pattern. See Reference 1. (*)

L1=25;    (*) Length in U1 direction (*)

L2=25;    (*) Length in U2 direction (*)

m=8;       (*) Frequency in U1 direction (*)

n=8;        (*) Frequency in U2 direction (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0]#

rinid(m,n,1,1)|[0,1,0; 1,0,0];

Grid=bb(L1/m,L2/n)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 9

(*) Formian scheme for configurations

similar to Fig 9:  Single layer grid with

ternate pattern. See Reference 1. (*)

L1=24;    (*) Length in U1 direction (*)

L2=21;    (*) Length in U2 direction (*)

m=8;       (*) Frequency in U1 direction (*)

n=8;        (*) Frequency in U2 direction,

n must be even (*)

E=rinid(m,n/2,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,1+n/2,2,2)|[0,0,0; 2,0,0]#

rinid(m-1,n/2,2,2)|[1,1,0; 3,1,0]#

rinid(2,n/2,2*m,2)|[0,0,0; 0,2,0]#

rinid(2,n,2*m,1)|[0,0,0; 0,1,0]#

rinid(2,n/2,2*m-1,2)|[0,1,0; 1,1,0];

Grid=bb(L1/(2*m),L2/n)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 10

(*) Formian scheme for configurations

similar to Fig 10:  Single layer grid with

ternate pattern and hexagonal openings.

See Reference 1. (*)

L1=36;    (*) Length in U1 direction (*)

L2=36;    (*) Length in U2 direction (*)

m=16;      (*) Frequency in U1 direction,

m must be a multiple of 3 plus 1 (*)

n=16;       (*) Frequency in U2 direction,

n must be a multiple of 4 (*)

mm=ric|((m-1)/3);  nn=ric|(N/4);

E=rinid(m,n/2,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,1+n/2,2,2)|[0,0,0; 2,0,0]#

rinid(m-1,n/2,2,2)|[1,1,0; 3,1,0]#

rinid(2,n,2*m,1)|[0,0,0; 0,1,0]#

rinid(2,n/2,2*m-1,2)|[0,1,0; 1,1,0];

RN=rinid(mm,nn,6,4)|[4,2,0];

Grid=bb(L1/(2*m),L2/n)|lux(RN)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 11

(*) Formian scheme for configurations

similar to Fig 11:  Single layer grid with

ternate pattern and hexagonal openings.

See Reference 1. (*)

L1=36;    (*) Length in U1 direction (*)

L2=36;    (*) Length in U2 direction (*)

m=16;      (*) Frequency in U1 direction,

m must be a multiple of 3 plus 1 (*)

n=16;       (*) Frequency in U2 direction,

n must be a multiple of 6 plus 4 (*)

mm=ric|((m-1)/3);  nn=ric|(N/4);

E=rinid(m,n/2,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,1+n/2,2,2)|[0,0,0; 2,0,0]#

rinid(m-1,n/2,2,2)|[1,1,0; 3,1,0]#

rinid(2,n,2*m,1)|[0,0,0; 0,1,0]#

rinid(2,n/2,2*m-1,2)|[0,1,0; 1,1,0];

RN=rinid(mm,nn,6,6)|[4,2,0]#

rinid(mm-1,nn-1,6,6)|[7,5,0];

Grid=bb(L1/(2*m),L2/n)|lux(RN)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 12

(*) Formian scheme for configurations

similar to Fig 12:  Single layer grid with

ternate pattern and triangular boundary.

See Reference 1. (*)

L=24;  (*) Length of each side of the grid (*)

m=9;    (*) Frequency (*)

Grid=bb(L/(2*m),L*(sqrt|3)/(2*m))|

genid(m,m,2,1,1,-1)|{[0,0,0; 2,0,0],

[1,1,0; 0,0,0], [1,1,0; 2,0,0]};

use &,vm(2),vt(2),vh(L/2,-L,3*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 13 و 14

(*) Formian scheme for configurations

similar to Figs 13 (n=3) and 14 (n=4):

Single layer grid with ternate pattern

and triangular boundary with cut

corners.  See Reference 1. (*)

L=30;    (*) Length of each side of

the original triangular grid (*)

m=12;   (*) Frequency of the

original triangular grid (*)

n=3;      (*) Size of corner cuts (*)

E=genid(m,m,2,1,1,-1)|{[0,0,0; 2,0,0],

[1,1,0; 0,0,0], [1,1,0; 2,0,0]};

RN=lam(1,m)|genid(n,n,2,1,1,-1)|[0,0,0]#

genid(1,n,2,-1,-1,1)|[m,m,0];

Grid=bb(L/(2*m),L*(sqrt|3)/(2*m))|

lux(RN)|E;

use &,vm(2),vt(2),vh(L/2,-L,3*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 15

(*) Formian scheme for configurations

similar to Fig 15:  Single layer grid with

ternate/hexagonal pattern and triangular

boundary.  See Reference 1. (*)

L=36;  (*) Length of each side of the grid (*)

m=17;  (*) Frequency, m must be odd (*)

p=ric|((m-1)/2);

E=genid(m,m,2,1,1,-1)|{[0,0,0; 2,0,0],

[1,1,0; 0,0,0], [1,1,0; 2,0,0]};

RN=genid(p,p,4,2,2,-1)|[3,1,0];

Grid=bb(L/(2*m),L*(sqrt|3)/(2*m))|

lux(RN)|E;

use &,vm(2),vt(2),vh(L/2,-L,3*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 16 و 17

(*) Formian scheme for configurations

similar to Figs 16 (n=5) and 17 (n=7):

Single layer grid with ternate/hexagonal

pattern and triangular boundary with cut

corners.  See Reference 1. (*)

L=40;   (*) Length of each side of

the original triangular grid (*)

m=21;  (*) Frequency of the original

triangular grid, m must be odd (*)

n=5;     (*) Size of corner cuts,

n must be odd (*)

p=ric|((m-1)/2);

E=genid(m,m,2,1,1,-1)|{[0,0,0; 2,0,0],

[1,1,0; 0,0,0], [1,1,0; 2,0,0]};

RN=genid(p,p,4,2,2,-1)|[3,1,0]#

lam(1,m)|genid(n,n,2,1,1,-1)|[0,0,0]#

genid(1,n,2,-1,-1,1)|[m,m,0];

Grid=bb(L/(2*m),L*(sqrt|3)/(2*m))|

lux(RN)|E;

use &,vm(2),vt(2),vh(L/2,-L,3*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 18، 19 و 20

(*) Formian scheme for configurations

similar to Fig 18 (m=6, n=5), Fig 19

(m=5, n=8) and Fig 20 (m=4, n=11):

Sectorate single layer grid with ternate

pattern. See Reference 1. (*)

L=20;  (*) Length of each side of the grid (*)

m=6;    (*) Frequency (*)

n=5;     (*) Number of sides (*)

E=bb(L/(2*m),L/(2*m*tan|(180/n)))|

genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

Grid=pex|rosad(0,0,n,360/n)|E;

use &,vm(2),vt(2),vh(0,-2*L,5*L,

0,0,0, 0,0,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 21، 22 و 23

(*) Formian scheme for configurations

similar to Fig 21 (m=13, n=5), 22

(m=11, n=7) and Fig 23 (m=10, n=7):

Sectorate single layer grid with

ternate/hexagonal pattern.

See Reference 1. (*)

L=20;  (*) Length of each side of the grid (*)

m=13;  (*) Frequency of the

original ternate grid (*)

n=5;     (*) Number of sides (*)

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

RN=genid(1,ric|((m+1)/2),4,2,-2,1)|[0,0,0];

Grid=rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|lux(RN)|E;

use &,vm(2),vt(2),vh(0,-2*L,5*L,

0,0,0, 0,0,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 24 و 25

(*) Formian scheme for configurations

similar to Fig 24 (m=9, n=6) and

Fig 25 (m=9, n=9):  Sectorate single

layer grid with ternate/hexagonal pattern.

See Reference 1. (*)

L=20;  (*) Length of each side of the grid (*)

m=9;   (*) Frequency of the original ternate

grid, m must be a multiple of 3 (*)

n=6;     (*) Number of sides (*)

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

RN=genid(1,((m/3)+1),6,3,-3,1)|[0,0,0];

Grid=pex|rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|lux(RN)|E;

use &,vm(2),vt(2),vh(0,-2*L,5*L,

0,0,0, 0,0,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 26 و 27

(*) Formian scheme for configurations

similar to Fig 26 (m=11, n=5) and

Fig 27 (m=11, n=6):  Sectorate single

layer grid with ternate/hexagonal pattern.

See Reference 1. (*)

L=20;  (*) Length of each side of the grid (*)

m=11;  (*) Frequency of the original ternate

grid, m must be a multiple of 3 minus 1(*)

n=5;     (*) Number of sides (*)

q=ric|((m+1)/3);

E=[-1,1,0; 1,1,0]#lam(1,0)|{[-1,1,0; 0,2,0],

[-2,2,0; 0,2,0]};

F=genid(1,q,6,3,-3,1)|E#

genid(1,q-1,6,3,-3,1)|(lamid(0,3)|lam(1,-1)|

[-2,2,0; -1,3,0]#ref(2,3)|E);

Grid=rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|F;

use &,vm(2),vt(2),vh(0,-2*L,5*L,

0,0,0, 0,0,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 28، 29 و 30

(*) Formian scheme for configurations

similar to Fig 28 (n=6), Fig 29 (n=8)

and Fig 30 (n=11):  Sectorate single layer

grid with hexagonal pattern.

See Reference 1. (*)

L=20;  (*) Length of each side of the grid (*)

m=9;    (*) Frequency of the original ternate

grid, m must be a multiple of 3 (*)

n=6;     (*) Number of sides (*)

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

RN=genid(1,((m/3)+1),6,3,-3,1)|[0,0,0]#

genid(1,m/3,6,3,-3,1)|[0,2,0]#

genid(1,((m/3)-1),6,3,-3,1)|[0,4,0];

Grid=pex|rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|lux(RN)|E;

use &,vm(2),vt(2),vh(0,-2*L,5*L,

0,0,0, 0,0,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 31

(*) Formian scheme for configurations

similar to Fig 31:  Single layer grid with

hexagonal (honycomb) pattern.

See Reference 1. (*)

L1=25;   (*) Length in U1 direction (*)

L2=24;   (*) Length in U2 direction (*)

m=4;      (*) Frequency of second row

of hegxagons in U1 direction (*)

n=5;       (*) Frequency of hegxagons in

U2 direction along the sides (*)

E=rinid(m,n,6,2)|lamid(2,2)|[1,1,0; 0,2,0]#

rinid(m,n+1,6,2)|[1,1,0; 3,1,0]#

rin(1,m-1,6)|lamid(5,n+1)|[4,0,0; 3,1,0]#

rinid(m-1,n+2,6,2)|[4,0,0; 6,0,0];

Grid=bb(L1/(6*m-2),L2/(2*(n+1)))|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 32

(*) Formian scheme for configurations

similar to Fig 32:  Single layer grid with

quadnate pattern. See Reference 1. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=6;     (*) Frequency of crosses

in U1 direction (*)

n=6;      (*) Frequency of crosses

in U2 direction (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 33

(*) Formian scheme for configurations

similar to Fig 33:  Single layer grid with

quadnate pattern and openings.

See Reference 1. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=13;     (*) Frequency of crosses in U1

direction, m must be a multiple

of 3 plus 1 (*)

n=13;     (*) Frequency of crosses in U2

direction, n must be a multiple

of 3 plus 1 (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

RN=rinid((m-1)/3,(n-1)/3,6,6)|[4,4,0];

Grid=bb(L1/(2*m),L2/(2*n))|lux(RN)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 34

(*) Formian scheme for configurations

similar to Fig 34:  Single layer grid with

quadnate pattern and openings.

See Reference 1. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=12;    (*) Frequency of crosses in U1

direction, m must be a multiple of 4 (*)

n=12;     (*) Frequency of crosses in U2

direction, n must be a multiple of 4 (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

RN=rinid(m/4,n/4,8,8)|[4,4,0]#

rinid((m/4)-1,(n/4)-1,8,8)|[8,8,0];

Grid=bb(L1/(2*m),L2/(2*n))|lux(RN)|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 35

(*) Formian scheme for configurations

similar to Fig 35:  Single layer grid with

quadnate pattern. See Reference 1. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=5;     (*) Frequency of crosses

in U1 direction (*)

n=5;      (*) Frequency of crosses

in U2 direction (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(2*m,2*n+1,1,1)|[0,0,0; 1,0,0]#

rinid(2*m+1,2*n,1,1)|[0,0,0; 0,1,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 36

(*) Formian scheme for configurations

similar to Fig 36:  Single layer grid with

quadnate pattern with openings.

See Reference 1. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=7;     (*) Frequency of crosses in U1

direction, m must be odd (*)

n=7;      (*) Frequency of crosses in U2

direction, n must be odd (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(2*m,2*n+1,1,1)|[0,0,0; 1,0,0]#

rinid(2*m+1,2*n,1,1)|[0,0,0; 0,1,0];

RN=rinid((m-1)/2,(n-1)/2,4,4)|[3,3,0];

Grid=bb(L1/(2*m),L2/(2*n))|

(lux(RN)|E#rinid((m-1)/2,(n-1)/2,4,4)|

rosad(3,3)|[3,2,0; 2,3,0]);

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 37

(*) Formian scheme for configurations

similar to Fig 37:  Double layer grid with

binate on binate pattern. See Section 1.7

of Reference 1. (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=6;       (*) Frequency in U1 direction (*)

n=6;        (*) Frequency in U2 direction (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(m-1,n,2,2)|[1,1,0; 3,1,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,0];

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/3,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 38

(*) Formian scheme for configurations

similar to Fig 38:  Double layer grid with

binate on binate pattern and some openings.

See Section 1.7 of Reference 1. (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;        (*) Depth of grid (*)

m=7;        (*) Frequency in

U1 direction, m must be odd (*)

n=7;         (*) Frequency in

U2 direction, n must be odd (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(m-1,n,2,2)|[1,1,0; 3,1,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,0];

RN=lamid(m,n)|[m-2,n-2,0];

G=T#lux(RN)|(B#W);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/6,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 39

(*) Formian scheme for configurations

similar to Fig 39:  Double layer grid with

binate on larger binate pattern.

See Section 1.7 of Reference 1. (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;        (*) Depth of grid (*)

m=7;        (*) Frequency in

U1 direction, m must be odd (*)

n=7;         (*) Frequency in

U2 direction, n must be odd (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(m-1,n,2,2)|[1,1,0; 3,1,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,0];

RN=rinid(tic|(m/2),tic|(n/2),4,4)|[3,3,0];

G=T#lux(RN)|(B#W);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/6,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 40

(*) Formian scheme for configurations

similar to Fig 40:  Mansard double layer

grid with binate on binate pattern.

See Section 1.7 of Reference 1. (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=6;       (*) Frequency in U1 direction (*)

n=6;        (*) Frequency in U2 direction (*)

T=rinid(m-1,n,2,2)|[1,1,1; 3,1,1]#

rinid(m,n-1,2,2)|[1,1,1; 1,3,1];

B=rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,1];

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/3,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 41

(*) Formian scheme for configurations

similar to Fig 41:  Mansard double layer

grid with binate on binate pattern and

some openings.

See Section 1.7 of Reference 1. (*)

L1=24;    (*) Length in U1 direction (*)

L2=24;    (*) Length in U2 direction (*)

D=2;        (*) Depth of grid (*)

m=8;        (*) Frequency in

U1 direction, m must be even (*)

n=8;         (*) Frequency in

U2 direction, n must be even (*)

T=rinid(m-1,n,2,2)|[1,1,1; 3,1,1]#

rinid(m,n-1,2,2)|[1,1,1; 1,3,1];

B=rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,1];

RN=lamid(m,n)|[m-4,n-4,0]#[m,n,0];

G=T#lux(RN)|(B#W);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/6,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 42

(*) Formian scheme for configurations

similar to Fig 42:  Mansard double layer

grid with binate on larger binate pattern.

See Section 1.7 of Reference 1. (*)

L1=24;    (*) Length in U1 direction (*)

L2=24;    (*) Length in U2 direction (*)

D=2;        (*) Depth of grid (*)

m=8;        (*) Frequency in

U1 direction, m must be even (*)

n=8;         (*) Frequency in

U2 direction, n must be even (*)

T=rinid(m-1,n,2,2)|[1,1,1; 3,1,1]#

rinid(m,n-1,2,2)|[1,1,1; 1,3,1];

B=rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,1];

RN=rinid((m-2)/2,(n-2)/2,4,4)|[4,4,0];

G=T#lux(RN)|(B#W);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/6,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 43

(*) Formian scheme for configurations

similar to Fig 43:  Double layer grid with

obnate on obnate pattern.

See Reference 1 (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=5;       (*) Frequency in U1 direction (*)

n=5;        (*) Frequency in U2 direction (*)

T=rinid(m,n,2,2)|lamid(1,1)|[1,0,1; 0,1,1]#

rinid(m-1,2,2,2*n)|[1,0,1; 3,0,1]#

rinid(2,n-1,2*m,2)|[0,1,1; 0,3,1];

B=rinid(m-1,n-1,2,2)|lamid(2,2)|

[1,1,0; 2,2,0]#rinid(m-1,2,2,2*(n-1))|

[1,1,0; 3,1,0]#rinid(2,n-1,2*(m-1),2)|

[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|rosad(1,1)|[1,0,1; 1,1,0]#

rinid(m-1,n-1,2,2)|rosad(2,2)|

[2,1,1; 2,2,0];

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/3,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 44

(*) Formian scheme for configurations

similar to Fig 44:  Double layer grid with

obnate on larger obnate pattern.

See Reference 1 (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=6;       (*) Frequency in

U1 direction, m must be even (*)

n=6;        (*) Frequency in

U2 direction, n must be even (*)

T=rinid(m,n,2,2)|lamid(1,1)|[1,0,1; 0,1,1]#

rinid(m-1,2,2,2*n)|[1,0,1; 3,0,1]#

rinid(2,n-1,2*m,2)|[0,1,1; 0,3,1];

B=rinid(m-1,n-1,2,2)|lamid(2,2)|

[1,1,0; 2,2,0]#rinid(m-1,2,2,2*(n-1))|

[1,1,0; 3,1,0]#rinid(2,n-1,2*(m-1),2)|

[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|rosad(1,1)|

[1,0,1; 1,1,0]#rinid(m-1,n-1,2,2)|

rosad(2,2)|[2,1,1; 2,2,0];

RN=rinid(ric|((m-2)/2),ric|((n-2)/2),4,4)|

rosad(4,4)|[4,2,0];

G=T#lux(RN)|(B#W);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/5,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 45

(*) Formian scheme for configurations

similar to Fig 45:  Mansard double layer

grid with obnate on obnate pattern.

See Reference 1 (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=5;       (*) Frequency in U1 direction (*)

n=5;        (*) Frequency in U2 direction (*)

T=rinid(m-1,n-1,2,2)|lamid(2,2)|

[1,1,1; 2,2,1]#rinid(m-1,2,2,2*(n-1))|

[1,1,1; 3,1,1]#rinid(2,n-1,2*(m-1),2)|

[1,1,1; 1,3,1];

B=rinid(m,n,2,2)|lamid(1,1)|[1,0,0; 0,1,0]#

rinid(m-1,2,2,2*n)|[1,0,0; 3,0,0]#

rinid(2,n-1,2*m,2)|[0,1,0; 0,3,0];

W=rinid(m,n,2,2)|rosad(1,1)|[1,0,0; 1,1,1]#

rinid(m-1,n-1,2,2)|

rosad(2,2)|[2,1,0; 2,2,1];

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/3,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 46

(*) Formian scheme for configurations

similar to Fig 46:  Mansard double layer

grid with obnate on larger obnate pattern.

See Reference 1 (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=7;       (*) Frequency in U1 direction (*)

n=7;        (*) Frequency in U2 direction (*)

T=rinid(m-1,n-1,2,2)|lamid(2,2)|

[1,1,1; 2,2,1]#rinid(m-1,2,2,2*(n-1))|

[1,1,1; 3,1,1]#rinid(2,n-1,2*(m-1),2)|

[1,1,1; 1,3,1];

B=rinid(m,n,2,2)|lamid(1,1)|[1,0,0; 0,1,0]#

rinid(m-1,2,2,2*n)|[1,0,0; 3,0,0]#

rinid(2,n-1,2*m,2)|[0,1,0; 0,3,0];

W=rinid(m,n,2,2)|rosad(1,1)|[1,0,0; 1,1,1]#

rinid(m-1,n-1,2,2)|

rosad(2,2)|[2,1,0; 2,2,1];

RN={[m,n-3,0], [m-3,n,0], [m,n+3,0],

[m+3,n,0]};

G=T#lux(RN)|(B#W);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/5,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 47

(*) Formian scheme for configurations

similar to Fig 47:  Double layer grid with

binate on larger obnate pattern.

See Reference 1 (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=7;       (*) Frequency in

U1 direction, m must be odd (*)

n=7;        (*) Frequency in

U2 direction, n must be odd (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(tic|(m/2),tic|(n/2),4,4)|lamid(3,3)|

[1,1,0; 3,3,0]#rinid(m-1,2,2,2*(n-1))|

[1,1,0; 3,1,0]#rinid(2,n-1,2*(m-1),2)|

[1,1,0; 1,3,0];

RN=rinid(ric|((m-3)/2),ric|((n-1)/2),4,4)|

[5,3,0]#rinid(ric|((m-1)/2),ric|

((n-3)/2),4,4)|[3,5,0];

W=lux(RN)|rinid(m,n,2,2)|

lamid(1,1)|[0,0,1; 1,1,0];

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 48

(*) Formian scheme for configurations

similar to Fig 48:  Double layer grid with

obnate on larger binate pattern.

See Reference 1 (*)

L1=20;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=5;       (*) Frequency in

U1 direction, m must be odd (*)

n=5;        (*) Frequency in

U2 direction, n must be odd (*)

T=rinid(m,n,2,2)|lamid(1,1)|[1,0,1; 0,1,1]#

rinid(m-1,2,2,2*n)|[1,0,1; 3,0,1]#

rinid(2,n-1,2*m,2)|[0,1,1; 0,3,1];

B=rinid(m-1,n,2,2)|[1,1,0; 3,1,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|rosad(1,1)|[1,0,1; 1,1,0];

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 49

(*) Formian scheme for configurations

similar to Fig 49:  Double layer grid with

ternate on ternate pattern.

See Reference 1 (*)

L1=24;    (*) Length in U1 direction (*)

L2=22;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=5;       (*) Frequency in U1 direction (*)

n=6;        (*) Frequency in

U2 direction, n must be even (*)

T=rinid(m,n/2,2,6)|lamid(2,3)|[1,0,1; 2,3,1]#

rinid(m,1+n/2,2,6)|[1,0,1; 3,0,1]#

rinid(m-1,n/2,2,6)|[2,3,1; 4,3,1];

B=rinid(m+1,n/2,2,6)|lamid(1,4)|

[0,1,0; 1,4,0]#rinid(m+1,1+n/2,2,6)|

[0,1,0; 2,1,0]#rinid(m,n/2,2,6)|

[1,4,0; 3,4,0];

W=rinid(m+1,1+n/2,2,6)|lam(1,1)|

[0,1,0; 1,0,1]#rinid(m,n/2,2,6)|

lam(1,2)|[1,4,0; 2,3,1]#

rinid(m+1,n/2,2,6)|[1,4,0; 1,6,1]#

rinid(m,n/2,2,6)|[2,1,0; 2,3,1];

Grid=bt(L1/(2*m+2),L2/(3*n+1),D)|

(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/2,4*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 50

(*) Formian scheme for configurations

similar to Fig 50:  Double layer grid with

hexagon on hexagon pattern.

See Reference 1 (*)

L1=22;    (*) Length in U1 direction (*)

L2=20;    (*) Length in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=5;       (*) Frequency in U1 direction (*)

n=3;        (*) Frequency in U2 direction (*)

T=rinid(m+1,n,4,12)|lamid(1,6)|

[0,3,1; 1,6,1]#rinid(m,n,4,12)|lamid(3,6)|

[2,3,1; 3,0,1]#rinid(2*m+1,2*n,2,6)|

[0,3,1; 2,3,1];

B=rinid(m,n,4,12)|lamid(3,6)|[1,4,0; 3,2,0]#

rinid(m+1,n,4,12)|[1,4,0; 1,8,0]#

rinid(m,n-1,4,12)|[3,10,0; 3,14,0];

W=rinid(m+1,n,4,12)|(lamid(1,6)|

[0,3,1; 1,4,0]#lam(2,6)|[1,6,1; 1,4,0])#

rinid(m,n,4,12)|(lamid(3,6)|[2,3,1; 3,2,0]#

lam(2,6)|[3,0,1; 3,2,0]);

Grid=bt(L1/(4*m+2),L2/(12*n),D)|

(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2,6*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 51

(*) Formian scheme for configurations

similar to Fig 51:  Double layer grid with

hexagon on triangle pattern.

See Reference 1 (*)

L1=22;   (*) Length in U1 direction (*)

L2=20;   (*) Length in U2 direction (*)

D=2;      (*) Depth of grid (*)

m=4;      (*) Frequency in U1 direction,

number of hexagons in the

second row of top layer (*)

n=5;       (*) Frequency in U2 direction,

number of hexagons along

the sides of top layer (*)

T=rinid(m,n,6,2)|lamid(2,2)|[1,1,1; 0,2,1]#

rin(1,m-1,6)|lamid(5,n+1)|[3,1,1; 4,0,1]#

rinid(m,n+1,6,2)|[1,1,1; 3,1,1]#

rinid(m-1,n+2,6,2)|[4,0,1; 6,0,1];

B=rinid(m-1,n,6,2)|lamid(5,2)|[2,2,0; 5,1,0]#

rinid(m,n-1,6,2)|[2,2,0; 2,4,0]#

rinid(m-1,n,6,2)|[5,1,0; 5,3,0];

W=rinid(m,n,6,2)|(lamid(2,2)|[1,1,1; 2,2,0]#

lam(1,2)|[0,2,1; 2,2,0])#

rinid(m-1,n+1,6,2)|(lamid(5,1)|

[4,0,1; 5,1,0]#lam(1,5)|[3,1,1; 5,1,0]);

Grid=bt(L1/(6*m-2),L2/((n+1)*2),D)|

(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/2,4*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 52

(*) Formian scheme for configurations

similar to Fig 52:  Double layer grid with

hexagon on larger hexagon pattern.

See Reference 1 (*)

L1=24;   (*) Length in U1 direction (*)

L2=21;   (*) Length in U2 direction (*)

D=2;      (*) Depth of grid (*)

m=5;      (*) Frequency in U1 direction (*)

n=6;       (*) Frequency in U2 direction,

n must be even (*)

T=rinid(m,n,6,2)|lamid(2,2)|[1,1,1; 0,2,1]#

rin(1,m-1,6)|lamid(5,n+1)|[3,1,1; 4,0,1]#

rinid(m,n+1,6,2)|[1,1,1; 3,1,1]#

rinid(m-1,n+2,6,2)|[4,0,1; 6,0,1];

RN=rinid(m-1,n/2,6,4)|[5,3,0];

B=lux(RN)|(rinid(m-1,n,6,2)|lamid(5,2)|

[2,2,0; 5,1,0]#rinid(m,n-1,6,2)|[2,2,0;

2,4,0]#rinid(m-1,n,6,2)|[5,1,0; 5,3,0]);

W=lux(RN)|(rinid(m,n,6,2)|(lamid(2,2)|

[1,1,1; 2,2,0]#lam(1,2)|[0,2,1; 2,2,0])#

rinid(m-1,n+1,6,2)|(lamid(5,1)|

[4,0,1; 5,1,0]#lam(1,5)|[3,1,1; 5,1,0]));

Grid=bt(L1/(6*m-2),L2/((n+1)*2),D)|

(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/2,4*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 53

(*) Formian scheme for configurations

similar to Fig 53:  Binate trussed double

layer grid. See Reference 1 (*)

L1=16;   (*) Length in U1 direction (*)

L2=16;   (*) Length in U2 direction (*)

D=1.5;    (*) Depth of grid (*)

m=5;       (*) Frequency in U1 direction (*)

n=5;        (*) Frequency in U2 direction (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(m,n+1,2,2)|[0,0,0; 2,0,0]#

rinid(m+1,n,2,2)|[0,0,0; 0,2,0];

E=lamis(1,0.5)|[0,0,0; 1,0,0.5]#

lam(1,1)|[0,0,0; 0,0,1];

W=pex|(rinid(m,n+1,2,2)|E#

rinid(m+1,n,2,2)|verad(0,0)|E);

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2/3,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 54

(*) Formian scheme for configurations

similar to Fig 54:  Obnate trussed

double layer grid. See Reference 1 (*)

L1=16;   (*) Length in U1 direction (*)

L2=16;   (*) Length in U2 direction (*)

D=1.5;   (*) Depth of grid (*)

m=4;      (*) Frequency in U1 direction (*)

n=4;       (*) Frequency in U2 direction (*)

T=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,1]#

rinid(m,2,2,2*n)|[0,0,1; 2,0,1]#

rinid(2,n,2*m,2)|[0,0,1; 0,2,1];

B=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,2,2,2*n)|[0,0,0; 2,0,0]#

rinid(2,n,2*m,2)|[0,0,0; 0,2,0];

E=lam(3,0.5)|{[0,0,0; 0.5,0.5,0.5],

[0.5,0.5,0.5; 1,1,0]}#

{[0,0,1; 0,0,0], [1,1,1; 1,1,0]};

F=lamis(1,0.5)|[0,0,0; 1,0,0.5]#

lam(1,1)|[0,0,0; 0,0,1];

W=pex|(rinid(m,n,2,2)|lamid(1,1)|E#

rinid(m,2,2,2*n)|F#rinid(2,n,2*m,2)|

verad(0,0)|F);

Grid=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

use &,vm(2),vt(2),vh(L1/2,-L2,2.5*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 55

(*) Formian scheme for configurations

similar to Fig 55:  Double layer grid with

ternate on ternate pattern and with a

triangular shape in plan.

See Section 1.7 of Reference 1. (*)

L=20;   (*) Length of each side of grid (*)

D=2;    (*) Depth of grid (*)

m=7;    (*) Number of tetrahedral units

along each side of the grid (*)

T=genid(m,m,2,3,1,-1)|([0,0,1; 2,0,1]#

lam(1,1)|[0,0,1; 1,3,1]);

B=genid(m-1,m-1,2,3,1,-1)|([1,1,0; 3,1,0]#

lam(1,2)|[1,1,0; 2,4,0]);

W=genid(m,m,2,3,1,-1)|([1,3,1; 1,1,0]#

lam(1,1)|[0,0,1; 1,1,0]);

Grid=bt(L/(2*m),L*sqrt|3/(6*m),D)|

(T#B#W);

use &,vm(2),vt(2),vh(L/2,-L/2,6*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به

شکلهای 56 و 57

(*) Formian scheme for configurations

similar to Fig 56 (m=12, n=3) and

Fig 57 (m=12, n=4):  Double layer grid

with ternate on ternate pattern and

a triangular plan with curtailed corners.

See Section 1.7 of Reference 1. (*)

L=40;    (*) Length of each side of the

complete triangular plan (*)

D=2;     (*) Depth of grid (*)

m=12;   (*) Number of tetrahedral units

along each side of the complete

triangular plan (*)

n=3;    (*) Extent of the corner

curtailment of the grid (*)

RN=lam(1,m)|genid(n,n,2,3,1,-1)|

{[0,0,1],[1,1,0]}#genid(1,n,2,-3,-1,1)|

{[m,3*m,1], [m,3*m-2,0]};

T=lux(RN)|genid(m,m,2,3,1,-1)|

([0,0,1; 2,0,1]#lam(1,1)|[0,0,1; 1,3,1]);

B=lux(RN)|genid(m-1,m-1,2,3,1,-1)|

([1,1,0; 3,1,0]#lam(1,2)|[1,1,0; 2,4,0]);

W=lux(RN)|genid(m,m,2,3,1,-1)|

([1,3,1; 1,1,0]#lam(1,1)|[0,0,1; 1,1,0]);

Grid=bt(L/(2*m),L*sqrt|3/(6*m),D)|

(T#B#W);

use &,vm(2),vt(2),vh(L/2,-L/2,6*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 58، 59 و 60

(*) Formian scheme for configurations

similar to Fig 58 (m=5, n=5), Fig 59

(m=4, n=6) and Fig 60 (m=4, n=8):

Sectorate double layer grid with ternate

on ternate pattern. See Section 2.2 of

Reference 2. (*)

L=20;      (*) Length of each side (*)

D=2.15;  (*) Depth of grid (*)

m=5;       (*) Frequency of top layer (*)

n=5;        (*) Number of sides (*)

S=L/(2*tan|(180/n));

ET=genid(1,m,2,3,-1,1)|{[0,0,0; -1,3,0],

[0,0,0; 1,3,0], [-1,3,0; 1,3,0]};

EB=genid(1,m-1,2,3,-1,1)|{[0,2,-1; -1,5,-1],

[0,2,-1; 1,5,-1], [-1,5,-1; 1,5,-1]};

EW=genid(1,m,2,3,-1,1)|{[0,0,0; 0,2,-1],

[-1,3,0; 0,2,-1], [1,3,0; 0,2,-1]};

E=bt(L/(2*m),S/(3*m),D)|(ET#EB#EW);

Qi=[0,2,-1];   Ri=bt(L/(2*m),S/(3*m),D)|Qi;

Rj=verad(0,0,360/n)|Ri;   K1=tig|(Ri#Rj);

K2=rinad(0,0,-L/(2*m),S/m,m)|K1;

Grid=pex|rosad(0,0,n,360/n)|(E#K2);

use &,vm(2);   clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 61، 62 و 63

(*) Formian scheme for configurations

similar to Fig 61 (n=5), Fig 62 (n=6)

and Fig 63 (n=7):  Sectorate double layer

grid with ternate/hexagonal on ternate/

hexagonal pattern. See Reference 1. (*)

L=20;  (*) Length of each side of the grid (*)

D=2;    (*) Depth of grid (*)

m=8;    (*) Frequency of the original ternate

pattern of the top layer, m must

be a multiple of 3 minus 1 (*)

n=5;     (*) Number of sides (*)

q=ric|((m+1)/3);

E=[-2,2,1; 2,2,1]#lam(1,0)|{[-2,2,1; 0,4,1],

[-4,4,1; 0,4,1]};

T=genid(1,q,12,6,-6,1)|E#

genid(1,q-1,12,6,-6,1)|(lamid(0,6)|

lam(1,-2)|[-4,4,1; -2,6,1]#ref(2,6)|E);

B=genid(1,q-1,12,6,-6,1)|lamid(0,6)|[-3,3,0;

0,6,0]#genid(1,q,6,6,-6,2)|[-3,3,0; 3,3,0];

W=(genid(1,q,12,6,-6,1)|lam(1,0)|

{[0,4,1; -3,3,0], [-2,2,1; -3,3,0],

[-4,4,1; -3,3,0]}#genid(1,q-1,12,6,-6,1)|

(lam(1,0)|{[-2,6,1; 0,6,0], [0,8,1; -3,9,0]}#

lam(2,6)|[0,4,1; 0,6,0]));

Grid=pex|rosad(0,0,n,360/n)|bt(L/(4*m),

L/(4*m*tan|(180/n)),D)|(T#B#W);

use &,vm(2);   clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به

شکلهای 64 و 65

(*) Formian scheme for configurations

similar to Fig 64 (p=6, q=6) and Fig 65

(p=4, q=2):  Compound double layer

grid with binate on binate pattern

(obtained by overlapping two grids).

See Section 1.7 of Reference 1. (*)

L1=16;    (*) Length of the base grid

in U1 direction (*)

L2=16;    (*) Length of the base grid

in U2 direction (*)

D=2;       (*) Depth of grid (*)

m=5;       (*) Frequency of the base grid

in U1 direction (*)

n=5;        (*) Frequency of the base grid

in U2 direction (*)

p=6;        (*) Shift in U1 direction,

p must be even (*)

q=6;        (*) Shift in U2 direction,

q must be even (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(m-1,n,2,2)|[1,1,0; 3,1,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,0];

G=T#B#W;

G=pex|(G#tranid(p,q)|G);

Grid=bt(L1/(2*m),L2/(2*n),D)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/3,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Grid;

برنامهء فرميينی برای تاشه های شبيه به شکل 66

(*) Formian scheme for configurations

similar to Fig 66:  Barrel vault with binate

pattern. See Section 1.8 of Reference 1. (*)

S=28;  (*) Span (*)

H=8;   (*) Rise (*)

L=37;  (*) Length (*)

m=9;   (*) Frequency in U2 direction (*)

n=12;  (*) Frequency in U3 direction (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n+1,1,1)|[1,0,0; 1,1,0]#

rinit(m+1,n,1,1)|[1,0,0; 1,0,1];

F=bc(R,2*A/m,L/n)|E;

Vault=verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(0,2.5*R,-R, 0,0,R, 0,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 67

(*) Formian scheme for configurations

similar to Fig 67:  Barrel vault with lamella

(obnate) pattern. See Section 1.8 of

Reference 1. (*)

S=30;   (*) Span (*)

H=8;    (*) Rise (*)

L=45;   (*) Length (*)

m=7;    (*) Frequency in U2 direction (*)

n=8;     (*) Frequency in U3 direction (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n,2,2)|lamit(1,1)|[1,0,0; 1,1,1]#

rinit(m,2,2,2*n)|[1,0,0; 1,2,0]#

rinit(2,n,2*m,2)|[1,0,0; 1,0,2];

F=bc(R,A/m,L/(2*n))|E;

Vault=verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(0,3*R,-2*R, 0,0,R, 0,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 68

(*) Formian scheme for configurations

similar to Fig 68:  Lamella barrel vault

with additional ribs.

See Section 1.8 of Reference 1. (*)

S=30;   (*) Span (*)

H=8;    (*) Rise (*)

L=45;   (*) Length (*)

m=7;    (*) Frequency in U2 direction (*)

n=8;     (*) Frequency in U3 direction (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n,2,2)|lamit(1,1)|[1,0,0; 1,1,1]#

rinit(m,n+1,2,2)|[1,0,0; 1,2,0]#

rinit(2,n,2*m,2)|[1,0,0; 1,0,2];

F=bc(R,A/m,L/(2*n))|E;

Vault=verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(0,3*R,-2*R, 0,0,R, 0,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 69

(*) Formian scheme for configurations

similar to Fig 69:  Binate barrel vault

with additional bracings.

See Section 1.8 of Reference 1. (*)

S=28;   (*) Span (*)

H=8;    (*) Rise (*)

L=32;   (*) Length (*)

m=8;    (*) Frequency

in U2 direction, m must be even (*)

n=10;   (*) Frequency

in U3 direction, n must be even (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

mm=ric|(m/2);   nn=ric|(n/2);

E=rinit(m,n+1,1,1)|[1,0,0; 1,1,0]#

rinit(m+1,n,1,1)|[1,0,0; 1,0,1]#

lamit(mm,nn)|rinit(mm,nn,1,1)|

[1,0,1; 1,1,0];

F=bc(R,2*A/m,L/n)|E;

Vault=verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(0,3*R,-R, 0,0,R, 0,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 70

(*) Formian scheme for configurations

similar to Fig 70:  Binate barrel vault

with additional bracings.

See Section 1.8 of Reference 1. (*)

S=28;   (*) Span (*)

H=8;    (*) Rise (*)

L=32;   (*) Length (*)

m=10;  (*) Frequency

in U2 direction, m must be even (*)

n=10;   (*) Frequency

in U3 direction, n must be even (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

mm=ric|(m/2);   nn=ric|(n/2);

E=lux(rin(3,nn,2)|[1,mm,1])|

(rinit(m,n+1,1,1)|[1,0,0; 1,1,0]#

rinit(m+1,n,1,1)|[1,0,0; 1,0,1]#

lamit(mm,nn)|rinit(mm,ric|(nn/2),1,2)|

lam(3,1)|[1,0,1; 1,1,0]);

F=bc(R,2*A/m,L/n)|E;

Vault=verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(0,3*R,-R, 0,0,R, 0,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 71

(*) Formian scheme for configurations

similar to Fig 71:  Double layer barrel

vault with binate on binate pattern.

See Section 1.8 of Reference 1. (*)

S=20;    (*) Span (of top layer) (*)

H=6;     (*) Rise (of top layer) (*)

D=1.5;  (*) Depth of the vault (*)

L=24;    (*) Length of the vault (*)

m=6;     (*) Frequency in U2 direction (*)

n=6;      (*) Frequency in U3 direction (*)

A=2*atan|(2*H/S);  (*) Sweep angle (*)

Rt=S/(2*sin|A);       (*) Top radius (*)

Rb=Rt-D;                (*) Bottom radius (*)

T=rinit(m,n+1,2,2)|[Rt,0,0; Rt,2,0]#

rinit(m+1,n,2,2)|[Rt,0,0; Rt,0,2];

B=rinit(m-1,n,2,2)|[Rb,1,1; Rb,3,1]#

rinit(m,n-1,2,2)|[Rb,1,1; Rb,1,3];

W=rinit(m,n,2,2)|lamit(1,1)|

[Rt,0,0; Rb,1,1];

E=bc(1,A/m,L/(2*n))|(T#B#W);

Vault=verad(0,0,90-A)|E;

use &,vm(2),vt(2),

vh(0,3*Rt,-Rt, 0,0,Rt, 0,1,Rt);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 72

(*) Formian scheme for configurations

similar to Fig 72:  Compound barrel vault.

See Section 1.8 of Reference 1. (*)

S=20;   (*) Span (*)

H=7;    (*) Rise (*)

L=36;   (*) Length (*)

m=6;    (*) Frequency in U2 direction (*)

n=8;     (*) Frequency in U3 direction (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n,2,2)|lamit(1,1)|[1,0,0; 1,1,1]#

rinit(m,n+1,2,2)|[1,0,0; 1,2,0]#

rinit(2,n,2*m,2)|[1,0,0; 1,0,2];

F=bc(R,A/m,L/(2*n))|E;

Vault=rin(1,2,S)|verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(S/2,6*R,-3*R, S/2,0,R, S/2,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 73

(*) Formian scheme for configurations

similar to Fig 73:  Compound barrel vault.

See Section 1.8 of Reference 1. (*)

S=16;   (*) Span of each barrel vault (*)

H=4;    (*) Rise (*)

L=40;   (*) Length (*)

m=6;    (*) Frequency

in U2 direction, m must be even (*)

n=10;   (*) Frequency

in U3 direction, n must be even (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

mm=ric|(m/2);   nn=ric|(n/2);

E=lux(rin(3,nn,2)|[1,mm,1])|

(rinit(m,n+1,1,1)|[1,0,0; 1,1,0]#

rinit(m+1,n,1,1)|[1,0,0; 1,0,1]#

lamit(mm,nn)|rinit(mm,ric|(nn/2),1,2)|

lam(3,1)|[1,0,1; 1,1,0]);

F=bc(R,2*A/m,L/n)|E;

Vault=rin(1,3,S)|verad(0,0,90-A)|F;

use &,vm(2),vt(2),

vh(S,8*R,-4*R, S,0,R, S,1,R);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 74

(*) Formian scheme for configurations

similar to Fig 74:  Sidebent annular barrel

vault with binate pattern.

See Section 3.4 of Reference 3. (*)

S=24;   (*) Span of barrel vault (*)

H=6;    (*) Rise of barrel vault (*)

m=20;  (*) Frequency in U2 direction (*)

n=10;    (*) Frequency in U4 direction (*)

R=60;   (*) Radius of annulus (*)

T=60;   (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n+1,1,1)|[1,0,1,-n/2;

1,1,1,-n/2]#rinic(2,4,m+1,n,1,1)|

[1,0,1,-n/2; 1,0,1,1-n/2];

Vault=dep(4)|ba(R,T/m,Q,2*A/n)|E;

use &,vm(2),vt(2),

vh(R/2,2*R,1.5*R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 75

(*) Formian scheme for configurations

similar to Fig 75:  Sidebent annular barrel

vault with lamella (obnate) pattern.

See Section 3.4 of Reference 3. (*)

S=28;   (*) Span of barrel vault (*)

H=8;    (*) Rise of barrel vault (*)

m=16;  (*) Frequency in U2 direction (*)

n=8;    (*) Frequency in U4 direction (*)

R=70;   (*) Radius of annulus (*)

T=50;   (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n,2,2)|lam(4,1-n)|lam(2,1)|

[1,0,1,-n; 1,1,1,1-n]#

rinic(2,4,m,2,2,2*n)|

[1,0,1,-n; 1,2,1,-n]#

rinic(2,4,2,n,2*m,2)|

[1,0,1,-n; 1,0,1,2-n];

Vault=dep(4)|ba(R,T/(2*m),Q,A/n)|E;

use &,vm(2),vt(2),

vh(R,2*R,2*R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 76

(*) Formian scheme for configurations

similar to Fig 76:  Sidebent horseshoe

annular barrel vault with binate pattern.

See Section 3.4 of Reference 3. (*)

S=60;    (*) Span of barrel vault (*)

H=15;   (*) Rise of barrel vault (*)

m=48;   (*) Frequency in U2 direction (*)

n=7;     (*) Frequency in U4 direction (*)

R=90;   (*) Radius of annulus (*)

T=270; (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n+1,1,1)|

[1,0,1,-n/2; 1,1,1,-n/2]#

rinic(2,4,m+1,n,1,1)|

[1,0,1,-n/2; 1,0,1,1-n/2];

Vault=dep(4)|ba(R,T/m,Q,2*A/n)|E;

use &,vm(2),vt(2),

vh(2*R,2*R,4*R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 77

(*) Formian scheme for configurations

similar to Fig 77:  Sidebent annular ring

barrel vault with lamella (obnate) pattern.

See Section 3.4 of Reference 3. (*)

S=50;   (*) Span of barrel vault (*)

H=15;   (*) Rise of barrel vault (*)

m=30;   (*) Frequency in U2 direction (*)

n=5;      (*) Frequency in U4 direction (*)

R=80;   (*) Radius of annulus (*)

T=360; (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n,2,2)|lam(4,1-n)|lam(2,1)|

[1,0,1,-n; 1,1,1,1-n]#

rinic(2,4,m,2,2,2*n)|

[1,0,1,-n; 1,2,1,-n];

Vault=dep(4)|ba(R,T/(2*m),Q,A/n)|E;

use &,vm(2),vt(2),

vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 78

(*) Formian scheme for configurations

similar to Fig 78:  Outbent annular

barrel vault with lamella (obnate) pattern

and additional ribs.

See Section 3.4 of Reference 3. (*)

S=48;   (*) Span of barrel vault (*)

H=10;   (*) Rise of barrel vault (*)

m=12;   (*) Frequency in U2 direction (*)

n=5;      (*) Frequency in U4 direction (*)

R=50;   (*) Radius of annulus (*)

T=90;   (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n,2,2)|lam(4,1-n)|lam(2,1)|

[1,0,1,-n; 1,1,1,1-n]#

rinic(2,4,m,2,2,2*n)|

[1,0,1,-n; 1,2,1,-n]#

rinic(2,4,m+1,n,2,2)|

[1,0,1,-n; 1,0,1,2-n];

E=tran(4,n*90/A)|E;

Vault=dep(4)|ba(R,T/(2*m),Q,A/n)|E;

use &,vm(2),vt(2),

vh(5*R,10*R,5*R, 0,0,0, 1,1,0);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 79

(*) Formian scheme for configurations

similar to Fig 79:  Compound outbent

annular barrel vault with lamella pattern.

See Section 3.5 of Reference 3. (*)

S=48;     (*) Span of barrel vault (*)

H=12;    (*) Rise of barrel vault (*)

m=10;    (*) Frequency in U2 direction (*)

n=5;       (*) Frequency in U4 direction (*)

R=50;     (*) Radius of annulus (*)

T=90;     (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);    (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n,2,2)|lam(4,1-n)|lam(2,1)|

[1,0,1,-n; 1,1,1,1-n]#

rinic(2,4,m,2,2,2*n)|[1,0,1,-n; 1,2,1,-n]#

rinic(2,4,2,n,2*m,2)|[1,0,1,-n; 1,0,1,2-n];

E=tran(4,n*90/A)|E;

B=dep(4)|ba(R,T/(2*m),Q,A/n)|E;

Vault=rin(3,3,S)|B;

use &,vm(2),vt(2),

vh(5*R,10*R,5*R, 0,0,0, 1,1,0);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 80

(*) Formian scheme for configurations

similar to Fig 80:  Inbent annular barrel

vault with binate pattern.

See Section 3.4 of Reference 3. (*)

S=40;     (*) Span of barrel vault (*)

H=8;      (*) Rise of barrel vault (*)

m=16;    (*) Frequency in U2 direction (*)

n=7;       (*) Frequency in U4 direction (*)

R=100;  (*) Radius of annulus (*)

T=90;    (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n+1,1,1)|

[1,0,1,-n/2; 1,1,1,-n/2]#

rinic(2,4,m+1,n,1,1)|

[1,0,1,-n/2; 1,0,1,1-n/2];

F=tran(4,-n*90/(2*A))|E;

Vault=dep(4)|ba(R,T/m,Q,2*A/n)|F;

use &,vm(2),vt(2),

vh(1.5*R,1.5*R,R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 81

(*) Formian scheme for configurations

similar to Fig 81:  Compound inbent

annular barrel vault with binate pattern.

See Section 3.5 of Reference 3. (*)

S=40;     (*) Span of barrel vault (*)

H=8;      (*) Rise of barrel vault (*)

m=16;    (*) Frequency in U2 direction (*)

n=7;       (*) Frequency in U4 direction (*)

R=100;  (*) Radius of annulus (*)

T=90;    (*) Central angle

containing the barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n+1,1,1)|

[1,0,1,-n/2; 1,1,1,-n/2]#

rinic(2,4,m+1,n,1,1)|

[1,0,1,-n/2; 1,0,1,1-n/2];

F=tran(4,-n*90/(2*A))|E;

G=dep(4)|ba(R,T/m,Q,2*A/n)|F;

Vault=rin(3,3,S)|G;

use &,vm(2),vt(2),

vh(6*R,6*R,8*R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 82

(*) Formian scheme for configurations

similar to Fig 82:  Double layer inbent

annular barrel vault with binate on

binate pattern. See Section 3.4 of

Reference 3. (*)

S=45;     (*) Span of top layer (*)

H=6;      (*) Rise of top layer (*)

D=2;      (*) Depth of barrel vault (*)

m=8;  (*) Top layer frequency along U2 (*)

n=5;   (*) Top layer frequency along U4 (*)

R=100;  (*) Radius of annulus (*)

TT=70;    (*) Central angle containing the

top layer of barrel vault (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Qt=S/(2*sin|A);          (*) Radius of top

layer cross-section of annulus (*)

Qb=Qt-D;

T=rinic(2,4,m,n+1,2,2)|

[1,0,Qt,-n; 1,2,Qt,-n]#

rinic(2,4,m+1,n,2,2)|

[1,0,Qt,-n; 1,0,Qt,2-n];

B=rinic(2,4,m-1,n,2,2)|

[1,1,Qb,1-n; 1,3,Qb,1-n]#

rinic(2,4,m,n-1,2,2)|

[1,1,Qb,1-n; 1,1,Qb,3-n];

W=rinic(2,4,m,n,2,2)|lamic(2,4,1,1-n)|

[1,0,Qt,-n; 1,1,Qb,1-n];

E=tran(4,-2*n*90/(2*A))|(T#B#W);

Vault=dep(4)|ba(R,TT/(2*m),1,A/n)|E;

use &,vm(2),vt(2),

vh(2*R,1.5*R,R, 0,0,0, 0,0,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 83 و 84

(*) Formian scheme for configurations

similar to Fig 83 (R1=16, R2=10) and

Fig 84 (R1=10, R2=15):  Parabolic barrel

vault. See Section 2.3 of  Reference 2. (*)

R1=16;   (*) Radius at the ends (*)

R2=10;   (*) Radius at the middle (*)

L=36;     (*) Length of barrel vault (*)

P=60;     (*) Sweep angle (*)

m=12;    (*) Frequency in U2 direction (*)

n=20;     (*) Frequency in U3 direction (*)

a=4*(R1-R2)/L^2;   b=-a*L;

c=R1;   d=L/n;

E=rin(2,m,1)|lib(i=0,n)|

[a*(d*i)^2+b*d*i+c,0,i;

a*(d*i)^2+b*d*i+c,1,i]#

rin(2,m+1,1)|lib(i=0,n-1)|

[a*(d*i)^2+b*d*i+c,0,i;

a*(d*(i+1))^2+b*d*(i+1)+c,0,i+1];

Vault=verad(0,0,90-P)|bc(1,2*P/m,d)|E;

use &, vm(2),vt(2),

vh(L,3*L,-L, 0,0,L, 0,1,L);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 85 و 86

(*) Formian scheme for configurations

similar to Fig 85 (R1=16, R2=12) and

Fig 86 (R1=12, R2=16):

Hyperboloidal barrel vault.

See Section 3.3.2 of Reference 3. (*)

R1=16;   (*) Radius at the ends (*)

R2=11;   (*) Radius at the middle (*)

L=36;     (*) Length of barrel vault (*)

A=60;     (*) Sweep angle (*)

m=4;  (*) Circumferential semifrequency (*)

n=6;   (*) Longitudinal semifrequency (*)

P=(R1^2-R2^2)/(L/2)^2;    (*) Pitch (*)

E=pex|lamit(0,0)|rinit(m,n,1,1)|

(rosat(0.5,0.5)|[1,0,0; 1,1,0]#

[1,0,1; 1,1,0]);

F=bh(R2,A/m,L/(2*n),P)|E;

Vault=verad(0,0)|F;

use &, vm(2),vt(2),

vh(L,3*L,-L, 0,0,L, 0,1,L);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه

به شکل 87 و 88

(*) Formian scheme for configurations

similar to Fig 87 (C=3, parabolic) and

Fig 88 (C=4, Wedge-shaped): Pellevated

honeycomb grid.

See References 4, 5 and 6. (*)

S=20;     (*) Span (*)

L=24;     (*) Length (*)

H=7;      (*) Rise (*)

m=6;      (*) Circuferential frequency,

number of hexagons in the second row (*)

n=7;       (*) Longitudinal frequency,

number of hexagons along the sides (*)

C=3;      (*) Pellevation code (*)

E=rinid(m,n,6,2)|lamid(2,2)|[1,1,0; 0,2,0]#

rinid(m,n+1,6,2)|[1,1,0; 3,1,0]#

rin(1,m-1,6)|lamid(5,n+1)|[4,0,0; 3,1,0]#

rinid(m-1,n+2,6,2)|[4,0,0; 6,0,0];

Grid=bb(S/(6*m-2),L/(2*(n+1)))|E;

Vault=bapel(C,S/2,0,S/2,1,S,H)|Grid;

use &,vm(2),vt(2),vh(L/2,-2*L,3*L,

S/2,L/2,0, S/2,L/2,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه

به شکل 89 و 90

(*) Formian scheme for configurations

similar to Fig 89 (C=3, parabolic) and

Fig 90 (C=4, Wedge-shaped): Pellevated

trussed lamella double layer barrel vault.

See References 4, 5 and 6. (*)

S=20;     (*) Span (*)

L=20;     (*) Length (*)

D=2;      (*) Depth of grid (*)

H=6;      (*) Rise (*)

m=4;      (*) Circuferential frequency (*)

n=4;       (*) Longitudinal frequency (*)

C=3;      (*) Pellevation code (*)

T=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,1]#

rinid(m,2,2,2*n)|[0,0,1; 2,0,1]#

rinid(2,n,2*m,2)|[0,0,1; 0,2,1];

B=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,2,2,2*n)|[0,0,0; 2,0,0]#

rinid(2,n,2*m,2)|[0,0,0; 0,2,0];

E=lam(3,0.5)|{[0,0,0; 0.5,0.5,0.5],

[0.5,0.5,0.5; 1,1,0]}#

{[0,0,1; 0,0,0], [1,1,1; 1,1,0]};

F=lamis(1,0.5)|[0,0,0; 1,0,0.5]#

lam(1,1)|[0,0,0; 0,0,1];

W=pex|(rinid(m,n,2,2)|lamid(1,1)|E#

rinid(m,2,2,2*n)|F#rinid(2,n,2*m,2)|

verad(0,0)|F);

Grid=bt(S/(2*m),L/(2*n),D)|(T#B#W);

Vault=bapel(C,S/2,0,S/2,1,S,H)|Grid;

use &,vm(2),vt(2),vh(L/2,-2*L,4*L,

S/2,L/2,0, S/2,L/2,1);

clear;   draw Vault;

برنامهء فرميينی برای تاشه های شبيه به شکل 91

(*) Formian scheme for configurations

similar to Fig 91:  Ribbed dome.

See Section 1.9 of Reference 1. (*)

S=36;      (*) Span (*)

H=6;       (*) Rise (*)

m=24;     (*) Frequency of

elements on a ring (*)

n=9;   (*) Frequency of elements on a rib (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);   (*) Radius (*)

E=rinit(m,n-1,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2]}#rin(2,m,1)|{[1,0,0; 1,0,1],

[1,0,n; 1,1,n]};

Dome=bs(R,360/m,A/n)|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 92

(*) Formian scheme for configurations

similar to Fig 92:  Trimmed ribbed dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=6;      (*) Rise (*)

m=24;    (*) Frequency of elements

on a ring, must be even (*)

n=9;  (*) Frequency of elements on a rib (*)

A=2*atan|(2*H/S);       (*) Sweep angle (*)

R=S/(2*sin|A);   (*) Radius (*)   p=ric|(m/2);

E=rinit(m,n-1,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2]}#rin(2,m,1)|{[1,0,0; 1,0,1],

[1,0,n; 1,1,n]};

E=lux(rin(2,p,2)|[1,0,1])|E#

rin(2,p,2)|[1,1,1; 1,3,1];

Dome=bs(R,360/m,A/n)|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 93

(*) Formian scheme for configurations

similar to Fig 93:  Schwedler dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=6;       (*) Rise (*)

m=16;     (*) Frequency of

elements on a ring (*)

n=8;  (*) Frequency of elements on a rib (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n-1,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2], [1,0,1; 1,1,2]}#rin(2,m,1)|

{[1,0,0; 1,0,1], [1,0,n; 1,1,n]};

Dome=bs(R,360/m,A/n)|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 94

(*) Formian scheme for configurations

similar to Fig 94:  Trimmed Schwedler

dome. See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=6;       (*) Rise (*)

m=16;     (*) Frequency of

elements on a ring, must be even (*)

n=8;  (*) Frequency of elements on a rib (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);   (*) Radius (*)   p=ric|(m/2);

E=rinit(m,n-1,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2]}#rin(2,m,1)|{[1,0,0; 1,0,1],

[1,0,n; 1,1,n]};

E=lux(rin(2,p+1,2)|[1,0,1])|E#rin(2,p,2)|

[1,1,1; 1,3,1];

E=E#rin(2,p,2)|[1,1,1; 1,3,2]#

rinit(m,n-2,1,1)|[1,0,2; 1,1,3];

Dome=bs(R,360/m,A/n)|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 95

(*) Formian scheme for configurations

similar to Fig 95:  Schwedler dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=6;       (*) Rise (*)

m=16;     (*) Frequency of

elements on a ring, must be even (*)

n=7;  (*) Frequency of elements on a rib (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2]}#rin(2,m,1)|{[1,0,0; 1,0,1],

[1,0,n+1; 1,1,n+1]}#

rinit(ric|(m/2),n,2,1)|lam(2,1)|[1,0,1; 1,1,2];

Dome=bs(R,360/m,A/(n+1))|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 96

(*) Formian scheme for configurations

similar to Fig 96:  Schwedler dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=6;       (*) Rise (*)

m=18;     (*) Frequency of

elements on a ring, must be even (*)

n=9;        (*) Frequency of

elements on a rib, must be odd (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n-1,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2]}#rin(2,m,1)|{[1,0,0; 1,0,1],

[1,0,n; 1,1,n]}#rinit(ric|(m/2),tic|(n/2),2,2)|

lamit(1,2)|[1,1,1; 1,2,2];

Dome=bs(R,360/m,A/n)|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 97

(*) Formian scheme for configurations

similar to Fig 97:  Schwedler dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=6;       (*) Rise (*)

m=18;     (*) Frequency of

elements on a ring, must be even (*)

n=8;        (*) Frequency of

elements on a rib, must be even (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n-1,1,1)|{[1,0,1; 1,1,1],

[1,0,1; 1,0,2]}#rin(2,m,1)|{[1,0,0; 1,0,1],

[1,0,n; 1,1,n]}#rinit(ric|(m/2),ric|(n/2),2,2)|

{[1,1,1; 1,2,2], [1,1,2; 1,2,1]}#

rinit(ric|(m/2),ric|(n/2)-1,2,2)|

{[1,2,2; 1,3,3], [1,2,3; 1,3,2]};

Dome=bs(R,360/m,A/n)|E;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 98

(*) Formian scheme for configurations

similar to Fig 98:  Double layer ribbed

canopy dome.

See Section 1.9 of Reference 1. (*)

S=60;     (*) Span of top layer (*)

P=25;     (*) Gap diameter (*)

D=2;       (*) Depth (*)

A=40;     (*) Sweep angle (*)

m=24;     (*) Frequency of

elements on a ring (*)

n=3;  (*) Frequency of elements on a rib (*)

R=S/(2*sin|A);          (*) Top radius (*)

Rb=R-D;                  (*) Bottom radius (*)

G=asin|(P/(2*R));     (*) Gap angle (*)

i=(A-G)/(2*n);          (*) increment (*)

T=rinit(m,n+1,2,2*i)|[R,0,G; R,2,G]#

rinit(m,n,2,2*i)|[R,0,G; R,0,G+2*i];

B=rinit(m,n,2,2*i)|[Rb,1,G+i; Rb,3,G+i]#

rinit(m,n-1,2,2*i)|[Rb,1,G+i; Rb,1,G+3*i];

W=rinit(m,n,2,2*i)|lamit(1,G+i)|

[R,0,G; Rb,1,G+i];

Dome=bs(1,360/(2*m),1)|(T#B#W);

use &,vm(2),vt(2),vh(R,R,4*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 99

(*) Formian scheme for configurations

similar to Fig 99:  Lamella dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=5;       (*) Rise (*)

m=16;     (*) Frequency of

rhombuses in U2 direction (*)

n=5;        (*) Frequency of

rhombuses in U3 direction (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m,n,2,2)|lamit(0,1)|[1,0,0; 1,1,1];

Dome=pex|bs(R,360/(2*m),A/n)|E;

use &,vm(2),vt(2),vh(R,R,4*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 100

(*) Formian scheme for configurations

similar to Fig 100:  Lamella dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=5;       (*) Rise (*)

m=16;     (*) Frequency of

rhombuses in U2 direction (*)

n=5;        (*) Frequency of

rhombuses in U3 direction (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=pex|rinit(m,n,2,2)|(lamit(0,1)|

[1,0,0; 1,1,1]#[1,1,1; 1,-1,1]);

Dome=bs(R,360/(2*m),A/n)|E;

use &,vm(2),vt(2),vh(R,R,4*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 101

(*) Formian scheme for configurations

similar to Fig 101:  Trimmed lamella dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=4;       (*) Rise (*)

m=24;     (*) Frequency of rhombuses

in U2 direction, m must be even (*)

n=6;        (*) Frequency of rhombuses

in U3 direction, n must be larger than 3 (*)

p=ric|(m/2);

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E=rinit(m, n-3,2,2)|lamit(0,7)|[1,0,6; 1,1,7];

F=rinit(p,3,4,2)|lamit(0,1)|[1,0,0; 1,2,1];

G=E#F#rinit(m,2,2,2*(n-3))|[1,0,6; 1,2,6]#

rin(2,p,4)|[1,2,6; 1,2,5];

Dome=pex|bs(R,360/(2*m),A/n)|G;

use &,vm(2),vt(2),vh(R,R,4*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به

شکلهای 102 و 103

(*) Formian scheme for configurations

similar to Fig 102 (f=1.2) and

Fig 103 (f=1.4): Ovate dome.

See Section 1.9 of Reference 1. (*)

S=36;     (*) Span (*)

H=8;      (*) Rise (*)

f=1.2;     (*) Factor for

scaling of half the dome (*)

A=2*atan|(2*H/S);   (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Radius (*)

E1=rin(2,3,4)|{[1,0,0; 1,0,1], [1,0,1; 1,4,1],

[1,0,1; 1,0,2], [1,0,1; 1,2,2],

[1,2,2; 1,4,1]}#rin(2,6,2)|{[1,0,2; 1,2,2],

[1,0,2; 1,0,3], [1,0,2; 1,2,3]}#

rin(2,6,2)|{[1,0,3; 1,2,3], [1,0,3; 1,0,4],

[1,0,3; 1,1,4], [1,1,4; 1,2,3]};

E2=rinit(12,3,1,1)|{[1,0,4; 1,1,4],

[1,0,4; 1,0,5], [1,0,4; 1,1,5]};

E3=E1#E2#rin(2,12,1)|[1,0,7; 1,1,7];

E4=bs(R,360/24,A/7)|E3;

Dome=E4#bb(1,f)|refid (0,0)|E4;

use &,vm(2),vt(2),vh(R,0,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 104

(*) Formian scheme for configurations

similar to Fig 104:  Diamatic dome.

See Section 2.5 of Reference 2. (*)

R=36;      (*) Radius of circumsphere (*)

A=40;      (*) Sweep angle (*)

m=7;        (*) Frequency (*)

n=6;         (*) Number of sectors (*)

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=bd(R,360/n,A/m)|E;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 105

(*) Formian scheme for configurations

similar to Fig 105:  Diamatic dome.

See Section 2.5 of Reference 2. (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=8;        (*) Frequency (*)

n=7;         (*) Number of sectors (*)

E=genit(1,m,3,3,0,1)|{[1,0,0; 1,1,2],

[1,1,2; 1,0,3], [1,1,2; 1,3,3]};

F=bd(R,360/n,A/(3*m))|E;

Dome=rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 106

(*) Formian scheme for configurations

similar to Fig 106:  Diamatic dome.

See Section 2.5 of Reference 2. (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=7;        (*) Frequency (*)

n=7;         (*) Number of sectors (*)

E=genit(1,m,3,3,0,1)|{[1,2,4; 1,1,2],

[1,2,4; 1,1,5], [1,2,4; 1,4,5]}#

rin(3,m+1,3)|[1,1,2; 1,-1,2];

F=bd(R,360/n,A/(3*m))|E;

Dome=rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 107

(*) Formian scheme for configurations

similar to Fig 107:  Diamatic dome.

See Section 2.5 of Reference 2. (*)

R=36;      (*) Radius of circumsphere (*)

A=40;      (*) Sweep angle (*)

m=9;        (*) Frequency (*)

n=6;         (*) Number of sectors (*)

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

Hole={[1,2,4], [1,2,7], [1,5,7]};

F=bd(R,360/n,A/m)|lux(Hole)|E;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 108

(*) Formian scheme for configurations

similar to Fig 108:  Diamatic dome.

See Section 2.5 of Reference 2. (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=8;        (*) Frequency (*)

n=7;         (*) Number of sectors (*)

E=genit(1,m,3,3,0,1)|{[1,0,0; 1,1,2],

[1,1,2; 1,0,3], [1,1,2; 1,3,3]};

Hole=rin(2,5,3)|[1,6,24]#rin(2,2,3)|[1,9,21]#

{[1,10,20], [1,7,23], [1,16,23]};

F=bd(R,360/n,A/(3*m))|lux(Hole)|E;

Dome=rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به

شکلهای 109 و 110

(*) Formian scheme for configurations

similar to Fig 109 (f1=1.15, f2=0.85) and

Fig 110 (f1=1.35, f2=1.05):

Ovate diamatic dome.

See Section 2.5 of Reference 2. (*)

R=36;  (*) Initial radius of circumsphere (*)

A=50;  (*) Sweep angle (*)

m=8;    (*) Frequency (*)

f1=1.15;   (*) Scale factor for 1st half (*)

f2=0.85;   (*) Scale factor for 2nd half (*)

n1=4;   (*) Number of sectors in 1st half (*)

n2=3;   (*) Number of sectors in 1st half (*)

E=genit(1,m,3,3,0,1)|{[1,0,0; 1,1,2],

[1,1,2; 1,0,3], [1,1,2; 1,3,3]};

E1=bd(R,180/n1,A/(3*m))|E;

E2=bd(R,180/n2,A/(3*m))|E;

Dome=dil(2,f1)|rosad(0,0,n1,180/n1)|E1#

dil(2,f2)|ref(2,0)|rosad(0,0,n2,180/n2)|E2;

use &,vm(2),vt(2),vh(R,0,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 111

(*) Formian scheme for configurations

similar to Fig 111:  Double layer diamatic

dome. See Section 2.5 of Reference 2. (*)

R=36;     (*) Radius of top circumsphere (*)

Rb=34.5;  (*) Radius of

bottom circumsphere (*)

A=45;      (*) Sweep angle (*)

m=3;        (*) Frequency  of top layer (*)

n=6;         (*) Number of sectors (*)

T=genit(1,m,3,3,0,1)|{[R,0,0; R,0,3],

[R,0,0; R,3,3], [R,0,3; R,3,3]};

B=genit(1,m-1,3,3,0,1)|{[Rb,2,4; Rb,1,2],

[Rb,2,4; Rb,1,5], [Rb,2,4; Rb,4,5]}#

rin(3,m,3)|[Rb,1,2; Rb,-1,2];

W=genit(1,m,3,3,0,1)|{[Rb,1,2; R,0,0],

[Rb,1,2; R,0,3], [Rb,1,2; R,3,3]}#

genit(1,m-1,3,3,0,1)|{[Rb,2,4; R,3,6],

[Rb,2,4; R,0,3], [Rb,2,4; R,3,3]};

F=bd(1,360/n,A/(3*m))|(T#B#W);

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 112

(*) Formian scheme for configurations

similar to Fig 112:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=8;        (*) Frequency (*)

n=6;         (*) Number of sectors (*)

C=1;        (*) Code (*)

D=9;        (*) Amplitude (*)

P=0;         (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,2*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 113

(*) Formian scheme for configurations

similar to Fig 113:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=70;      (*) Sweep angle (*)

m=7;        (*) Frequency (*)

n=8;         (*) Number of sectors (*)

C=1;        (*) Code (*)

D=15;      (*) Amplitude (*)

P=0;         (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 114

(*) Formian scheme for configurations

similar to Fig 114:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=70;      (*) Sweep angle (*)

m=8;        (*) Frequency (*)

n=5;         (*) Number of sectors (*)

C=1;        (*) Code (*)

D=8;      (*) Amplitude (*)

P=15;       (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 115

(*) Formian scheme for configurations

similar to Fig 115:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=32;      (*) Sweep angle (*)

m=7;        (*) Frequency (*)

n=12;       (*) Number of sectors (*)

C=1;        (*) Code (*)

D=4;        (*) Amplitude (*)

P=0;        (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,2*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 116

(*) Formian scheme for configurations

similar to Fig 116:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=8;        (*) Frequency (*)

n=6;         (*) Number of sectors (*)

C=2;        (*) Code (*)

D=10;      (*) Amplitude (*)

P=0;         (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,2*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 117

(*) Formian scheme for configurations

similar to Fig 117:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=60;      (*) Sweep angle (*)

m=7;        (*) Frequency (*)

n=8;         (*) Number of sectors (*)

C=2;        (*) Code (*)

D=6;        (*) Amplitude (*)

P=0;        (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,2*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 118

(*) Formian scheme for configurations

similar to Fig 118:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=11;      (*) Frequency (*)

n=3;         (*) Number of sectors (*)

C=2;        (*) Code (*)

D=5;        (*) Amplitude (*)

P=20;        (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 119

(*) Formian scheme for configurations

similar to Fig 119:  Scallop dome.

See Reference 12. (*)

R=36;      (*) Radius of circumsphere (*)

A=70;      (*) Sweep angle (*)

m=10;      (*) Frequency (*)

n=2;         (*) Number of sectors (*)

C=2;        (*) Code (*)

D=8;        (*) Amplitude (*)

P=30;       (*) Prominence (*)

nn=360/n;

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=sbd(C,nn,[R,m,D,P],1,nn,A/m)|E;

Dome=pex|rosad(0,0,n,nn)|F;

use &,vm(2),vt(2),vh(R,R,4*R, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 120

(*) Formian scheme for configurations

similar to Fig 120:  Onion dome.

See Section 3.4 of Reference 3. (*)

R=10;       (*) First radius (*)

Q=50;       (*) Second radius (*)

B=40;       (*) Base angle (*)

m=24;   (*) Number of rhombi along U2 (*)

n=8;      (*) Number of rhombi along U3 (*)

V=1;     (*) Vertical scale factor (*)

A=acos|(R/Q)+B;      (*) Sweep angle (*)

P=asin|(R/Q);             (*) Position angle (*)

E=rinic(2,4,m,n,2,-2)|lamic(2,4,1,-1)|

[1,1,1,0; 1,0,1,-1]#rin(2,m,2)|

[1,1,1,-2*n; 1,3,1,-2*n];

F=tran(4,-2*n*P/A)|E;

G=pex|dep(4)|ba(R,360/(2*m),Q,A/(2*n))|F;

Dome=dil(3,V)|G;

use &,vm(2),vt(2),

vh(0,100000*Q,Q, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 121

(*) Formian scheme for configurations

similar to Fig 121:  Onion dome.

See Section 3.4 of Reference 3. (*)

R=10;      (*) First radius (*)

Q=50;      (*) Second radius (*)

B=20;       (*) Base angle (*)

m=18;   (*) Number of rhombi along U2 (*)

n=8;      (*) Number of rhombi along U3 (*)

V=2.5;  (*) Vertical scale factor (*)

A=acos|(R/Q)+B;      (*) Sweep angle (*)

P=asin|(R/Q);             (*) Position angle (*)

E=rinic(2,4,m,n,2,-2)|lamic(2,4,1,-1)|

[1,1,1,0; 1,0,1,-1]#rin(2,m,2)|

[1,1,1,-2*n; 1,3,1,-2*n];

F=tran(4,-2*n*P/A)|E;

G=pex|dep(4)|ba(R,360/(2*m),Q,A/(2*n))|F;

Dome=dil(3,V)|G;

use &,vm(2),vt(2),

vh(0,100000*Q,Q, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 122

(*) Formian scheme for configurations

similar to Fig 122:  Onion dome.

See Section 3.4 of Reference 3. (*)

R=10;      (*) First radius (*)

Q=65;      (*) Second radius (*)

B=60;      (*) Base angle (*)

m=20;   (*) Number of rhombi along U2 (*)

n=8;      (*) Number of rhombi along U3 (*)

V=2.5;  (*) Vertical scale factor (*)

A=acos|(R/Q)+B;      (*) Sweep angle (*)

P=asin|(R/Q);             (*) Position angle (*)

E=rinic(2,4,m,n,2,-2)|lamic(2,4,1,-1)|

[1,1,1,0; 1,0,1,-1]#rin(2,m,2)|

[1,1,1,-2*n; 1,3,1,-2*n];

F=tran(4,-2*n*P/A)|E;

G=pex|dep(4)|ba(R,360/(2*m),Q,A/(2*n))|F;

Dome=dil(3,V)|G;

use &,vm(2),vt(2),

vh(0,100000*Q,Q, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 123

(*) Formian scheme for configurations

similar to Fig 123:  Onion dome.

See Section 3.4 of Reference 3. (*)

R=10;      (*) First radius (*)

Q=50;      (*) Second radius (*)

B=40;      (*) Base angle (*)

m=24;   (*) Number of rhombi along U2 (*)

n=8;      (*) Number of rhombi along U3 (*)

V=0.5;  (*) Vertical scale factor (*)

A=acos|(R/Q)+B;      (*) Sweep angle (*)

P=asin|(R/Q);             (*) Position angle (*)

E=rinic(2,4,m,n,2,-2)|lamic(2,4,1,-1)|

[1,1,1,0; 1,0,1,-1]#rin(2,m,2)|

[1,1,1,-2*n; 1,3,1,-2*n];

F=tran(4,-2*n*P/A)|E;

G=pex|dep(4)|ba(R,360/(2*m),Q,A/(2*n))|F;

Dome=dil(3,V)|G;

use &,vm(2),vt(2),

vh(0,100000*Q,Q, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 124

(*) Formian scheme for configurations

similar to Fig 124: Upcurved mallow

dome. See Section 3.2 of Reference 3. (*)

L=40;        (*) Span of dome (*)

C=20;       (*) Central height (*)

P=0.04;     (*) Pitch (*)

T=-1;        (*) Curving style coefficient (*)

m=7;         (*) Frequency (*)

n=7;          (*) Number of sectors (*)

B=360/(2*n);

D=sqrt|(cos|B^2*L*L/4+C*C);

(*)  D must be lager than C  (*)

A=acos|(C/D);   S=D/m;

W=D*sin|A*tan|B;   K=W/m;

E1=genid(1,m,2,S,-1,1)|{[0,0,0; -1,S,0],

[0,0,0; 1,S,0], [-1,S,0; 1,S,0]};

E2=verat(0,0,A-90)|E1;

E3=dil(1,K)|E2;

E4=verad(0,0,T*45)|E3;

F=bhp(1,1,1,P)|E4;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),

vh(L,L,1.3*L, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 125

(*) Formian scheme for configurations

similar to Fig 125:  Upcurved mallow

dome. See Section 3.2 of Reference 3. (*)

L=40;        (*) Span of dome (*)

C=6;         (*) Central height (*)

P=0.04;     (*) Pitch (*)

T=-1;        (*) Curving style coefficient (*)

m=9;         (*) Frequency (*)

n=3;          (*) Number of sectors (*)

B=360/(2*n);

D=sqrt|(cos|B^2*L*L/4+C*C);

(*)  D must be lager than C  (*)

A=acos|(C/D);   S=D/m;

W=D*sin|A*tan|B;   K=W/m;

E1=genid(1,m,2,S,-1,1)|{[0,0,0; -1,S,0],

[0,0,0; 1,S,0], [-1,S,0; 1,S,0]};

E2=verat(0,0,A-90)|E1;

E3=dil(1,K)|E2;

E4=verad(0,0,T*45)|E3;

F=bhp(1,1,1,P)|E4;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(L,L,2*L, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 126

(*) Formian scheme for configurations

similar to Fig 126:  Upcurved mallow

dome. See Section 3.2 of Reference 3. (*)

L=36;        (*) Span of dome (*)

C=20;       (*) Central height (*)

P=0.05;     (*) Pitch (*)

T=-1;        (*) Curving style coefficient (*)

m=7;         (*) Frequency (*)

n=10;        (*) Number of sectors (*)

B=360/(2*n);

D=sqrt|(cos|B^2*L*L/4+C*C);

(*)  D must be lager than C  (*)

A=acos|(C/D);   S=D/m;

W=D*sin|A*tan|B;   K=W/m;

E1=genid(1,m,2,S,-1,1)|{[0,0,0; -1,S,0],

[0,0,0; 1,S,0], [-1,S,0; 1,S,0]};

E2=verat(0,0,A-90)|E1;

E3=dil(1,K)|E2;

E4=verad(0,0,T*45)|E3;

F=bhp(1,1,1,P)|E4;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),

vh(L,L,1.7*L, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 127

(*) Formian scheme for configurations

similar to Fig 127:  Downcurved mallow

dome. See Section 3.2 of Reference 3. (*)

L=36;        (*) Span of dome (*)

C=1.5;       (*) Central height (*)

P=0.03;     (*) Pitch (*)

T=1;          (*) Curving style coefficient (*)

m=7;          (*) Frequency (*)

n=8;           (*) Number of sectors (*)

B=360/(2*n);

D=sqrt|(cos|B^2*L*L/4+C*C);

(*)  D must be lager than C  (*)

A=acos|(C/D);   S=D/m;

W=D*sin|A*tan|B;   K=W/m;

E1=genid(1,m,2,S,-1,1)|{[0,0,0; -1,S,0],

[0,0,0; 1,S,0], [-1,S,0; 1,S,0]};

E2=verat(0,0,A-90)|E1;

E3=dil(1,K)|E2;

E4=verad(0,0,T*45)|E3;

F=bhp(1,1,1,P)|E4;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),vh(L,L,2*L, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 128 و 129

(*) Formian scheme for configurations

similar to Fig 128 (n=4) and Fig 129 (n=3):

Downcurved mallow dome.

See Section 3.2 of Reference 3. (*)

L=36;        (*) Span of dome (*)

C=4.5;       (*) Central height (*)

P=0.03;     (*) Pitch (*)

T=1;          (*) Curving style coefficient (*)

m=8;          (*) Frequency (*)

n=4;           (*) Number of sectors (*)

B=360/(2*n);

D=sqrt|(cos|B^2*L*L/4+C*C);

(*)  D must be lager than C  (*)

A=acos|(C/D);   S=D/m;

W=D*sin|A*tan|B;   K=W/m;

E1=genid(1,m,2,S,-1,1)|{[0,0,0; -1,S,0],

[0,0,0; 1,S,0], [-1,S,0; 1,S,0]};

E2=verat(0,0,A-90)|E1;

E3=dil(1,K)|E2;

E4=verad(0,0,T*45)|E3;

F=bhp(1,1,1,P)|E4;

Dome=pex|rosad(0,0,n,360/n)|F;

use &,vm(2),vt(2),

vh(L,3*L,6*L, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 130

(*) Formian scheme for configurations

similar to Fig 130:  Pellevated dome

(spherical) with binate pattern.

See References 4, 5 and 6. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=12;   (*) Frequency in U1 direction (*)

n=12;    (*) Frequency in U2 direction (*)

S=sqrt|(L1^2+L2^2);

(*) Span of pellevant (*)

R=0.3*S;         (*) Rise of pellevant (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

Grid=bb(L1/m,L2/n)|E;

Dome=capel(1,L1/2,L2/2,S,R)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 131

(*) Formian scheme for configurations

similar to Fig 131:  Pellevated dome

(parabolic) with obnate pattern.

See References 4, 5 and 6. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=7;     (*) Frequency of crosses

in U1 direction (*)

n=7;      (*) Frequency of crosses

in U2 direction (*)

S=sqrt|(L1^2+L2^2);

(*) Span of pellevant (*)

R=0.35*S;       (*) Rise of pellevant (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,2,2,2*n)|[0,0,0; 2,0,0]#

rinid(2,n,2*m,2)|[0,0,0; 0,2,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

Dome=capel(3,L1/2,L2/2,S,R)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 132

(*) Formian scheme for configurations

similar to Fig 132:  Compound pellevated

dome (parabolic) with obnate pattern.

See References 4, 5 and 6. (*)

L1=25;  (*) Length in U1 direction (*)

L2=25;  (*) Length in U2 direction (*)

m=6;     (*) Frequency of crosses

in U1 direction (*)

n=6;      (*) Frequency of crosses

in U2 direction (*)

S=sqrt|(L1^2+L2^2);

(*) Span of pellevant (*)

R=0.25*S;       (*) Rise of pellevant (*)

E=rinid(m,n,2,2)|lamid(1,1)|[0,0,0; 1,1,0]#

rinid(m,2,2,2*n)|[0,0,0; 2,0,0]#

rinid(2,n,2*m,2)|[0,0,0; 0,2,0];

Grid=bb(L1/(2*m),L2/(2*n))|E;

D=capel(3,L1/2,L2/2,S,R)|Grid;

Dome=pex|rinid(2,2,L1,L2)|D;

use &,vm(2),vt(2),vh(L1/2,-L2,3*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 133

(*) Formian scheme for configurations

similar to Fig 133:  Pellevated dome

(conical) with ternate pattern.

See References 4, 5 and 6. (*)

L=24;  (*) Length of each side of dome (*)

m=12;  (*) Frequency (*)

Grid=bb(L/(2*m),L*(sqrt|3)/(2*m))|

genid(m,m,2,1,1,-1)|{[0,0,0; 2,0,0],

[1,1,0; 0,0,0], [1,1,0; 2,0,0]};

Dome=capel(4,L/2,S/4,S,R)|Grid;

use &,vm(2),vt(2),vh(L/2,-L,2*L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 134

(*) Formian scheme for configurations

similar to Fig 134:  Pellevated sectorate

dome (parabolic).

See References 4, 5 and 6. (*)

S=40;        (*) Span of dome (*)

H=10;        (*) Rise (*)

m=7;          (*) Frequency (*)

n=6;           (*) Number of sides (*)

nn=180/n;   L=S*sin|nn;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

F=bt(L/(2*m),L/(2*m*tan|nn),1)|E;

Base=pex|rosad(0,0,n,360/n)|F;

Dome=capel(3,0,0,S,H)|Base;

use &,vm(2),vt(2),vh(0,S,2*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 135 و 136

(*) Formian scheme for configurations

similar to Fig 135 (n=5) and Fig 136 (n=6):

Pellevated sectorate dome (spherical) with

ternate/hexagonal pattern.

See References 4, 5 and 6. (*)

L=20;  (*) Length of each side of the grid (*)

m=11;  (*) Frequency of the original ternate

grid, m must be a multiple of 3 minus 1(*)

n=5;     (*) Number of sides (*)

S=L/(sin|(360/(2*n)));

(*) Span of pellevant (*)

R=0.25*S;          (*) Rise of pellevant (*)

q=ric|((m+1)/3);

E=[-1,1,0; 1,1,0]#lam(1,0)|{[-1,1,0; 0,2,0],

[-2,2,0; 0,2,0]};

F=genid(1,q,6,3,-3,1)|E#

genid(1,q-1,6,3,-3,1)|(lamid(0,3)|lam(1,-1)|

[-2,2,0; -1,3,0]#ref(2,3)|E);

Grid=rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|F;

Dome=capel(1,0,0,S,R)|Grid;

use &,vm(2),vt(2),vh(0,-2*L,4*L,

0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه

به شکل 137

(*) Formian scheme for configurations

similar to Fig 137:  Pellevated sectorate

dome (parabolic) with honeycomb pattern.

See References 4, 5 and 6. (*)

L=20;  (*) Length of each side of the grid (*)

m=9;    (*) Frequency of the original ternate

grid, m must be a multiple of 3 (*)

n=8;     (*) Number of sides (*)

S=L/(sin|(360/(2*n)));

(*) Span of pellevant (*)

R=0.35*S;          (*) Rise of pellevant (*)

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

RN=genid(1,((m/3)+1),6,3,-3,1)|[0,0,0]#

genid(1,m/3,6,3,-3,1)|[0,2,0]#

genid(1,((m/3)-1),6,3,-3,1)|[0,4,0];

Grid=pex|rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|lux(RN)|E;

Dome=capel(3,0,0,S,R)|Grid;

use &,vm(2),vt(2),vh(0,-2*L,5*L,

0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 138

(*) Formian scheme for configurations

similar to Fig 138:  Pellevated sectorate

dome (tri-spherical). See References

4, 5 and 6. (*)

S=40;        (*) Span of dome (*)

H=4;         (*) Rise (*)

m=7;         (*) Frequency (*)

n=6;          (*) Number of sides (*)

nn=180/n;   L=S*sin|nn;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

F=bt(L/(2*m),L/(2*m*tan|nn),1)|E;

Base=pex|rosad(0,0,n,360/n)|F;

Dome=capel(1,S*sin|nn/4,S*cos|nn/4,S,H)|

capel(1,S*sin|nn/4,-S*cos|nn/4,S,H)|

capel(1,-S*sin|nn/4,-S*cos|nn/4,S,H)|Base;

use &,vm(2),vt(2),vh(0,S,S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 139

(*) Formian scheme for configurations

similar to Fig 139:  Pellevated dome

(parabolic + conical). See References

4, 5 and 6. (*)

S=40;     (*) Span of dome (*)

H1=6;     (*) Rise of the paraboloid (*)

H2=4;     (*) Rise of the cone (*)

m=6;       (*) Frequency (*)

n=8;        (*) Number of sides (*)

nn=180/n;   L=S*sin|nn;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

F=bt(L/(2*m),L/(2*m*tan|nn),1)|E;

Base=pex|rosad(0,0,n,360/n)|F;

Dome=capel(3,0,0,S,H1)|

capel(4,0,0,S,H2)|Base;

use &,vm(2),vt(2),

vh(S,0,1.4*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 140

(*) Formian scheme for configurations

similar to Fig 140:  Pellevated double layer

dome (spherical). See References

4, 5 and 6. (*)

L1=20;     (*) Length in U1 direction (*)

L2=20;     (*) Length in U2 direction (*)

D=1.5;      (*) Depth in U3 direction (*)

H=6;         (*) Rise of dome (*)

m=6;   (*) Frequency in U1 direction (*)

n=6;    (*) Frequency in U2 direction (*)

T=rinid(m,n+1,2,2)|[0,0,1; 2,0,1]#

rinid(m+1,n,2,2)|[0,0,1; 0,2,1];

B=rinid(m-1,n,2,2)|[1,1,0; 3,1,0]#

rinid(m,n-1,2,2)|[1,1,0; 1,3,0];

W=rinid(m,n,2,2)|lamid(1,1)|[0,0,1; 1,1,0];

G=bt(L1/(2*m),L2/(2*n),D)|(T#B#W);

Dome=capel(1,L1/2,L2/2,

sqrt|(L1^2+L2^2),H)|G;

use &,vm(2),vt(2),vh(L1/2,-L2/2,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 141

(*) Formian scheme for configurations

similar to Fig 141:  Pellevated double layer

dome (parabolic): See References

4, 5 and 6. (*)

L=20;     (*) Length of each side in plan (*)

H=8;      (*) Rise of dome (*)

D=2.15; (*) Depth of grid (*)

m=3;      (*) Frequency of top layer (*)

n=8;       (*) Number of sides (*)

S=L/tan|(180/n);  (*) Span (*)

ET=genid(1,m,2,3,-1,1)|{[0,0,0; -1,3,0],

[0,0,0; 1,3,0], [-1,3,0; 1,3,0]};

EB=genid(1,m-1,2,3,-1,1)|{[0,2,-1; -1,5,-1],

[0,2,-1; 1,5,-1], [-1,5,-1; 1,5,-1]};

EW=genid(1,m,2,3,-1,1)|{[0,0,0; 0,2,-1],

[-1,3,0; 0,2,-1], [1,3,0; 0,2,-1]};

E=bt(L/(2*m),S/(6*m),D)|(ET#EB#EW);

Qi=[0,2,-1];   Ri=bt(L/(2*m),S/(6*m),D)|Qi;

Rj=verad(0,0,360/n)|Ri;   K1=tig|(Ri#Rj);

K2=rinad(0,0,-L/(2*m),S/(2*m),m)|K1;

Grid=pex|rosad(0,0,n,360/n)|(E#K2);

Dome=capel(3,0,0,S,H)|Grid;

use &,vm(2),vt(2),vh(0,S,2*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 142

(*) Formian scheme for configurations

similar to Fig 142:  Geodesic dome,

based on top 5 faces of icosahedron.

See References 7 and 8. (*)

S=60;   (*) Span (*)

m=7;    (*) Frequency of subdivision (*)

R=sqrt|(10+2*sqrt|5)*sin|36*S/4;

E=pol(1,5,'[1-5]',R,[0,0; 2*m,0])|

dil(2,sqrt|3)|genid(m,m,2,1,1,-1)|

{[0,0; 2,0], [0,0; 1,1], [2,0; 1,1]};

Dome=trac(1,[0,0,0,R],4,-15)|E;

use &,vm(2),vt(2),vh(S,S,3*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 143

(*) Formian scheme for configurations

similar to Fig 143:  Geodesic dome,

based on top 5 faces of icosahedron.

See References 7 and 8. (*)

S=60;   (*) Span (*)

m=12;  (*) Frequency,

m must be divisible by 3 (*)

mm=m/3-1;

R=sqrt|(10+2*sqrt|5)*sin|36*S/4;

E=genid(m,m,2,1,1,-1)|{[0,0; 2,0],

[0,0; 1,1], [2,0; 1,1]};

E=lux(genid(mm,mm,6,3,3,-1)|[6,2])|E;

G=pol(1,5,'[1-5]',R,[0,0; 2*m,0])|

dil(2,sqrt|3)|E;

Dome=trac(1,[0,0,0,R],4,-15)|G;

use &,vm(2),vt(2),vh(S,S,3*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 144

(*) Formian scheme for configurations

similar to Fig 144:  Geodesic dome,

based on top 5 faces of icosahedron.

See References 7 and 8. (*)

S=60;   (*) Span (*)

m=14;  (*) Frequency, m must be even (*)

p=tic|(m/2)+1;

R=sqrt|(10+2*sqrt|5)*sin|36*S/4;

E=genid(m,m,2,1,1,-1)|{[0,0; 2,0],

[0,0; 1,1], [2,0; 1,1]};

C=genid(p,p,4,2,2,-1)|[0,0];

G=pol(1,5,'[1-5]',R,[0,0; 2*m,0])|

dil(2,sqrt|3)|lux(C)|E;

Dome=trac(1,[0,0,0,R],4,-15)|G;

use &,vm(2),vt(2),vh(S,S,3*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 145

(*) Formian scheme for configurations

similar to Fig 145:  Geodesic dome,

based on top 5 faces of icosahedron.

See References 7 and 8. (*)

S=60;   (*) Span (*)

m=14;   (*) Frequency,

m must be 5 plus a multiple of 3 (*)

p=(m-2)/3;

R=sqrt|(10+2*sqrt|5)*sin|36*S/4;

E=genid(m,m,2,1,1,-1)|{[0,0; 2,0],

[0,0; 1,1], [2,0; 1,1]};

C=[0,0]#[2*m,0]#genid(p,p,6,3,3,-1)|

[5,1]#genid(p+1,p+1,6,3,3,-1)|

{[2,0], [2,2]};

G=pol(1,5,'[1-5]',R,[0,0; 2*m,0])|

dil(2,sqrt|3)|lux(C)|E;

Dome=trac(1,[0,0,0,R],4,-15)|G;

use &,vm(2),vt(2),vh(S,S,3*S, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 146

(*) Formian scheme for configurations

similar to Fig 146:  Geodesic dome of

Fig 145, scaled vertically to make it

ellipsoidal. See References 7 and 8. (*)

S=60;   (*) Span (*)

m=14;   (*) Frequency,

m must be 5 plus a multiple of 3 (*)

f=1.75;    (*) Scale factor (*)

p=(m-2)/3;

R=sqrt|(10+2*sqrt|5)*sin|36*S/4;

E=genid(m,m,2,1,1,-1)|{[0,0; 2,0],

[0,0; 1,1], [2,0; 1,1]};

C=[0,0]#[2*m,0]#genid(p,p,6,3,3,-1)|

[5,1]#genid(p+1,p+1,6,3,3,-1)|

{[2,0], [2,2]};

G=pol(1,5,'[1-5]',R,[0,0; 2*m,0])|

dil(2,sqrt|3)|lux(C)|E;

Dome=dil(3,f)|trac(1,[0,0,0,R],4,-15)|G;

use &,vm(2),vt(2),

vh(0,100000*S,0, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 147

(*) Formian scheme for configurations

similar to Fig 147:  Geodesic dome of

Fig 145, pellevated to add a conical

effect. See References 7 and 8. (*)

S=60;    (*) Span (*)

H=12;   (*) Pellevational rise (*)

m=14;   (*) Frequency,

m must be 5 plus a multiple of 3 (*)

p=(m-2)/3;

R=sqrt|(10+2*sqrt|5)*sin|36*S/4;

E=genid(m,m,2,1,1,-1)|{[0,0; 2,0],

[0,0; 1,1], [2,0; 1,1]};

C=[0,0]#[2*m,0]#genid(p,p,6,3,3,-1)|[5,1]#

genid(p+1,p+1,6,3,3,-1)|{[2,0], [2,2]};

G=pol(1,5,'[1-5]',R,[0,0; 2*m,0])|

dil(2,sqrt|3)|lux(C)|E;

D=trac(1,[0,0,0,R],4,-15)|G;

Dome=capel(4,0,0,S,H)|D;

use &,vm(2),vt(2),

vh(0,100000*S,0, 0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 148 و 149

(*) Formian scheme for configurations

similar to Figs 148 (P=2) and 149

(P=4):  Novated dome with binate

pattern and edges held in position

(novation mode 3 is used).

See References 9, 10 and 11. (*)

L1=45;    (*) Length in U1 direction (*)

L2=45;    (*) Length in U2 direction (*)

m=18;     (*) Frequency in U1 direction (*)

n=18;      (*) Frequency in U2 direction (*)

Sh=11;    (*) Specifed height (*)

P=2;        (*) Position specifier (*)

Sb=[L1/P,L2/P,0];    (*) Specified base (*)

St=[L1/P,L2/P,Sh];   (*) Specified target (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

Grid=bb(L1/m,L2/n)|E;

Edge=bb(L1/m,L2/n)|(rinid(m+1,2,1,n)|

[0,0,0]#rinid(2,n-1,m,1)|[0,1,0]);

Dome=nov(3,1,Edge#Sb,Edge#St)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,1.5*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 150

(*) Formian scheme for configurations

similar to Fig 150:  Novated dome with

binate pattern and edges held in position

(novation mode 2 is used).

See References 9, 10 and 11. (*)

L1=45;    (*) Length in U1 direction (*)

L2=45;    (*) Length in U2 direction (*)

m=18;     (*) Frequency in U1 direction (*)

n=18;      (*) Frequency in U2 direction (*)

Sh=12;    (*) Specifed height (*)

Sb=[L1/2,L2/2,0];    (*) Specified base (*)

St=[L1/2,L2/2,Sh];   (*) Specified target (*)

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

Grid=bb(L1/m,L2/n)|E;

Edge=bb(L1/m,L2/n)|(rinid(m+1,2,1,n)|

[0,0,0]#rinid(2,n-1,m,1)|[0,1,0]);

Dome=nov(2,1,Edge#Sb,Edge#St)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,1.5*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 151

(*) Formian scheme for configurations

similar to Fig 151:  Novated dome

with binate pattern, four raised points and

edges held in position (novation mode 3 is

used). See References 9, 10 and 11. (*)

L1=45;    (*) Length in U1 direction (*)

L2=45;    (*) Length in U2 direction (*)

m=18;     (*) Frequency in U1 direction (*)

n=18;      (*) Frequency in U2 direction (*)

Sh=5;      (*) Specifed height (*)

Sb=rinid(2,2,L1/2,L2/2)|[L1/4,L2/4,0];

St=proj(3,Sh)|Sb;

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

Grid=bb(L1/m,L2/n)|E;

Edge=bb(L1/m,L2/n)|(rinid(m+1,2,1,n)|

[0,0,0]#rinid(2,n-1,m,1)|[0,1,0]);

Dome=nov(3,1,Edge#Sb,Edge#St)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,1.5*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 152، 153 و 154

(*) Formian scheme for configurations

similar to Figs 152 (n=4), 153 (n=5)

and 154 (n=6):  Sectorate novated dome

with ternate pattern, a central raised point

and with corners held in position.

See References 9, 10 and 11. (*)

L=30;  (*) Length of each side (*)

m=7;    (*) Frequency (*)

n=4;     (*) Number of sides (*)

Sh=7;   (*) Specified height (*)

E=bb(L/(2*m),L/(2*m*tan|(180/n)))|

genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

Grid=pex|rosad(0,0,n,360/n)|E;

Corner=rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|[m,m,0];

Dome=nov(3,1,Corner#[0,0,0],

Corner#[0,0,Sh])|Grid;

use &,vm(2),vt(2),vh(0,-2*L,1.5*L,

0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 155

(*) Formian scheme for configurations

similar to Fig 155:  Novated dome with

ternate pattern and triangular boundary

and with a central raised point and corners

held in position.

See References 9, 10 and 11. (*)

L=36;     (*) Length of each side (*)

m=12;    (*) Frequency (*)

Sh=L/7;  (*) Specifed height (*)

s=L*(sqrt|3)/2;

Corner=rosad(L/2,s/3,3,120)|[0,0,0];

Grid=bb(L/(2*m),s/m)|

genid(m,m,2,1,1,-1)|{[0,0,0; 2,0,0],

[1,1,0; 0,0,0], [1,1,0; 2,0,0]};

Dome=nov(3,1,Corner#[L/2,s/3,0],Corner#

[L/2,s/3,Sh])|Grid;

use &,vm(2),vt(2),vh(L/2,-L,L,

L/2,L/2,0, L/2,L/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 156

(*) Formian scheme for configurations

similar to Fig 156:  Novated dome

with binate pattern, a number of raised

points and with edges held in position.

See References 9, 10 and 11. (*)

L1=45;    (*) Length in U1 direction (*)

L2=45;    (*) Length in U2 direction (*)

m=27;     (*) Frequency in U1 direction,

must be a multiple of 3 (*)

n=27;      (*) Frequency in U2 direction,

must be a multiple of 3 (*)

m1=m/3;   n1=n/3;

Sh=3.5;   (*) Specifed height (*)

Sb=bb(L1/m,L2/n)|(rinid(2*m1+1,2,1,2*n1)|

[m1/2,n1/2,0]#rinid(2,2*n1-1,2*m1,1)|

[m1/2,1+n1/2,0]);

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

E=lux(rinid(m1-1,n1-1,1,1)|[m1+1,n1+1,0])|E;

Grid=bb(L1/m,L2/n)|E;

Edge=bb(L1/m,L2/n)|(rinid(m+1,2,1,n)|

[0,0,0]#rinid(2,n-1,m,1)|[0,1,0]#

rinid(m1+1,2,1,n1)|[m1,n1,0]#

rinid(2,n1-1,m1,1)|[m1,n1+1,0]);

Dome=nov(3,1,Edge#Sb,Edge#

proj(3,Sh)|Sb)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,2*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 157 و 158

(*) Formian scheme for configurations

similar to Figs 157 (n=5) and 158

(n=6):  Sectorate novated dome with

ternate pattern, n raised points and with

outside corners and inside edges held in

position. See References 9, 10 and 11. (*)

L=30;   (*) Length of each side (*)

m=9;     (*) Frequency (*)

n=5;      (*) Number of sides (*)

Sh=2.5; (*) Specified height (*)

Sb=rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|[m/2,m/2,0];

E=bb(L/(2*m),L/(2*m*tan|(180/n)))|

lux({[0,0,0], [1,1,0], [-1,1,0]})|

genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

Grid=pex|rosad(0,0,n,360/n)|E;

Corner=rosad(0,0,n,360/n)|bb(L/(2*m),

L/(2*m*tan|(180/n)))|

{[m,m,0], [2,2,0], [0,2,0]};

Dome=nov(3,1,Corner#Sb,

Corner#proj(3,Sh)|Sb)|Grid;

use &,vm(2),vt(2),vh(0,-2*L,1.5*L,

0,0,0, 0,0,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 159

(*) Formian scheme for configurations

similar to Fig 159:  Novated dome

with binate pattern, three raised points

and edges held in position.

See References 9, 10 and 11. (*)

L1=45;    (*) Length in U1 direction (*)

L2=45;    (*) Length in U2 direction (*)

m=24;     (*) Frequency in U1 direction,

must be a multiple of 3 (*)

n=24;      (*) Frequency in U2 direction,

must be a multiple of 3 (*)

Sh=6;      (*) Specifed height (*)

m1=m/3;  n1=n/3;  L11=L1/3;  L21=L2/3;

Sb={[L11,2*L21,0], [2*L11,2*L21,0],

[2*L11,L21,0]};

E=rinid(m,n+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,n,1,1)|[0,0,0; 0,1,0];

E=lux(rinid(m1+1,n1+1,1,1)|[0,0,0])|E;

Grid=bb(L1/m,L2/n)|E;

Edge=bb(L1/m,L2/n)|

(rinid(m1+1,2,1,2*n1-1)|

[0,n1+1,0]#rinid(2*m1,2,1,n)|[m1+1,0,0]#

rinid(2,n1+1,2*m1-1,1)|[m1+1,1,0]#

rinid(2,2*m1-2,m,1)|[0,n1+2,0]);

Dome=nov(3,1,Edge#Sb,Edge#

proj(3,Sh)|Sb)|Grid;

use &,vm(2),vt(2),vh(L1/2,-L2,1.5*L2,

L1/2,L2/2,0, L1/2,L2/2,1);

clear;   draw Dome;

برنامهء فرميينی برای تاشه های شبيه به شکل 160

(*) Formian scheme for configurations

similar to Fig 160:  Barrel-dome.

See Section 3.5 of Reference 3. (*)

S=40; (*) Span of barrel vault

and the semi-domes (*)

H=12; (*) Rise of barrel vault

and the semi domes (*)

L=32; (*) Length of barrel vault (*)

m=6;  (*) Frequency of the semi-domes (*)

n=8;   (*) Frequency along the

length of barrel vault, n must be even (*)

p=3;  (*) Number of sectors of

each semi-dome (*)

A=2*atan|(2*H/S);     (*) Sweep angle (*)

R=S/(2*sin|A);           (*) Circumradius (*)

E=rinit(2*m+1,n,1,1)|[1,0,0; 1,0,1]#

rinit(2*m,n-1,1,1)|[1,0,1; 1,1,1]#

lamit(m,n/2)|rinit(m,n/2,1,1)|

[1,1,0; 1,0,1];

B=verad(0,0,90-A)|bc(R,A/m,L/n)|E;

F=bd(R,180/p,A/m)|genit(1,m,1,1,0,1)|

{[1,0,0; 1,0,1], [1,0,0; 1,1,1],

[1,0,1; 1,1,1]};

D=pex|rosad(0,0,p,180/p)|F;

Domes=lam(2,-L/2)|D;

Barrel=verat(0,0)|B;

Bard=Barrel#Domes;

use &,vm(2),vt(2),

vh(2*R,2*R,8*R, 0,0,0, 0,0,1);

clear;   draw Bard;

برنامهء فرميينی برای تاشه های شبيه به شکل 161

(*) Formian scheme for configurations

similar to Fig 161:  Barrel-dome.

See Section 3.5 of Reference 3. (*)

S=35; (*) Span of barrel vault

and the semi-domes (*)

H=10; (*) Rise of barrel vault

and the semi domes (*)

L=20; (*) Length of barrel vault (*)

m=6;  (*) Frequency of the semi-domes (*)

n=6;   (*) Frequency along the length of

barrel vault, n must be even (*)

p=3;  (*) Number of sectors of

each semi-dome (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

R=S/(2*sin|A);          (*) Circumradius (*)

E=rinit(2*m+1,n,1,1)|[1,0,0; 1,0,1]#

rinit(2*m,n-1,1,1)|[1,0,1; 1,1,1]#

lamit(m,n/2)|rinit(m,n/2,1,1)|

[1,0,0; 1,1,1];

B=verad(0,0,90-A)|bc(R,A/m,L/n)|E;

F=bd(R,180/p,A/m)|genit(1,m,1,1,0,1)|

{[1,0,0; 1,0,1], [1,0,0; 1,1,1],

[1,0,1; 1,1,1]};

D=pex|rosad(0,0,p,180/p)|F;

Domes=lam(2,-L/2)|D;

Barrel=verat(0,0)|B;

Bard=Barrel#Domes;

use &,vm(2),vt(2),

vh(2*R,2*R,8*R, 0,0,0, 0,0,1);

clear;   draw Bard;

برنامهء فرميينی برای تاشه های شبيه به شکل 162

(*) Formian scheme for configurations

similar to Fig 162:  Barrel-dome.

See Section 3.5 of Reference 3. (*)

S=35; (*) Span of barrel vault

and the semi-domes (*)

H=10; (*) Rise of barrel vault

and the semi domes (*)

L=25;  (*) Length of barrel vault (*)

m=6;   (*) Frequency of the semi-domes (*)

n=8;    (*) Frequency along the length

of barrel vault, n must be even (*)

p=3;    (*) Number of sectors of

each semi-dome (*)

A=2*atan|(2*H/S);      (*) Sweep angle (*)

R=S/(2*sin|A);            (*) Circumradius (*)

E=rinit(2*m+1,n,1,1)|[1,0,0; 1,0,1]#

rinit(2*m,n-1,1,1)|[1,0,1; 1,1,1]#

lamit(m,n/2)|rinit(m,n/2,1,2)|

lam(3,1)|[1,1,0; 1,0,1];

E=lux(rin(3,n/2,2)|[1,m,1])|E;

B=verad(0,0,90-A)|bc(R,A/m,L/n)|E;

F=bd(R,180/p,A/m)|genit(1,m,1,1,0,1)|

{[1,0,0; 1,0,1], [1,0,0; 1,1,1],

[1,0,1; 1,1,1]};

D=pex|rosad(0,0,p,180/p)|F;

Domes=lam(2,-L/2)|D;

Barrel=verat(0,0)|B;

Bard=Barrel#Domes;

use &,vm(2),vt(2),

vh(2*R,2*R,8*R, 0,0,0, 0,0,1);

clear;   draw Bard;

برنامهء فرميينی برای تاشه های شبيه به شکل 163

(*) Formian scheme for configurations

similar to Fig 163:  Barrel-dome.

See Section 3.5 of Reference 3. (*)

S=35;  (*) Span of barrel vault

and the semi-domes (*)

H=10;  (*) Rise of barrel vault

and the semi domes (*)

T=45;  (*) Central angle

containing the barrel vault (*)

R=60;  (*) Radius of annulus (*)

f=5;     (*) Frequency of the semi-domes (*)

m=10;  (*) Frequency along the length

of barrel vault, m must be even (*)

p=3;    (*) Number of sectors of

each semi-dome (*)

A=2*atan|(2*H/S);      (*) Sweep angle (*)

Q=S/(2*sin|A);            (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,2*f+1,1,1)|[1,0,1,-f;

1,1,1,-f]#rinic(2,4,m+1,2*f,1,1)|

[1,0,1,-f; 1,0,1,1-f]#

lamic(2,4,ric|(m/2),0)|rinic(2,4,ric|

(m/2),f,1,1)|[1,0,1,0; 1,1,1,1];

Barrel=dep(4)|ba(R,T/m,Q,A/f)|E;

F=bd(Q,180/p,A/f)|genit(1,f,1,1,0,1)|

{[1,0,0; 1,0,1], [1,0,0; 1,1,1],

[1,0,1; 1,1,1]};

D=tran(1,R)|rosad(0,0,p,180/p)|F;

Domes=ref(2,0)|D#verad(0,0,T)|D;

Bard=pex|(Barrel#Domes);

use &,vm(2),vt(2),

vh(2*S,4*S,6*S, 0,0,0, 0,0,1);

clear;   draw Bard;

برنامهء فرميينی برای تاشه های شبيه به شکل 164

(*) Formian scheme for configurations

similar to Fig 164:  Barrel-dome.

See Section 3.5 of Reference 3. (*)

S=35;  (*) Span of barrel vault

and the semi-domes (*)

H=10;  (*) Rise of barrel vault

and the semi domes (*)

T=90;  (*) Central angle

containing the barrel vault (*)

R=60; (*) Radius of annulus (*)

f=5;     (*) Frequency of the semi-domes (*)

m=20;  (*) Frequency along the length

of barrel vault, m must be even (*)

p=3;    (*) Number of sectors of

each semi-dome (*)

A=2*atan|(2*H/S);      (*) Sweep angle (*)

Q=S/(2*sin|A);            (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,2*f+1,1,1)|[1,0,1,-f;

1,1,1,-f]#rinic(2,4,m+1,2*f,1,1)|

[1,0,1,-f; 1,0,1,1-f]#

lamic(2,4,ric|(m/2),0)|rinic(2,4,ric|

(m/2),f,1,1)|[1,1,1,0; 1,0,1,1];

Barrel=dep(4)|ba(R,T/m,Q,A/f)|E;

F=bd(Q,180/p,A/f)|genit(1,f,1,1,0,1)|

{[1,0,0; 1,0,1], [1,0,0; 1,1,1],

[1,0,1; 1,1,1]};

D=tran(1,R)|rosad(0,0,p,180/p)|F;

Domes=ref(2,0)|D#verad(0,0,T)|D;

Bard=pex|(Barrel#Domes);

use &,vm(2),vt(2),

vh(0,-2*S,8*S, 0,0,0, 0,0,1);

clear;   draw Bard;

برنامهء فرميينی برای تاشه های شبيه به شکل 165

(*) Formian scheme for configurations

similar to Fig 165:  Barrel-dome.

See Section 3.5 of Reference 3. (*)

S=35;  (*) Span of barrel vault

and the semi-domes (*)

H=10;  (*) Rise of barrel vault

and the semi domes (*)

T=25;  (*) Central angle

containing half the barrel vault (*)

R=60; (*) Radius of annulus (*)

f=5;    (*) Frequency of the semi-domes (*)

m=6;  (*) Frequency along

half the length of barrel vault (*)

p=3;   (*) Number of sectors of

each semi-dome (*)

A=2*atan|(2*H/S);      (*) Sweep angle (*)

Q=S/(2*sin|A);            (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,2*f+1,1,1)|[1,0,1,-f;

1,1,1,-f]#rinic(2,4,m+1,2*f,1,1)|

[1,0,1,-f; 1,0,1,1-f]#

lam(4,0)|rinic(2,4,m,f,1,1)|

[1,0,1,0; 1,1,1,1];

B=dep(4)|ba(R,T/m,Q,A/f)|E;

F=bd(Q,180/p,A/f)|genit(1,f,1,1,0,1)|

{[1,0,0; 1,0,1], [1,0,0; 1,1,1],

[1,0,1; 1,1,1]};

D=tran(1,R)|rosad(0,0,p,180/p)|F;

BD=B#verad(0,0,T)|D;

Bard=pex|(BD#verad(R,0,180)|BD);

use &,vm(2),vt(2),

vh(3*S,4*S,8*S, 0,0,0, 0,0,1);

clear;   draw Bard;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 166 و 167

(*) Formian scheme for configurations

similar to Fig 166 (H=3) and Fig 167

(H=12):  Simple hypar.  See Section 3.2

of Reference 3. (*)

L=20;     (*) Length of side in plan (*)

H=3;       (*) Height (*)

m=6;       (*) Semi-frequency (*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

Hypar=bhp(L/n,L/n,1,P)|E;

use &,vm(2),vt(2),vh(L,-L,L,0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 168

(*) Formian scheme for configurations

similar to Fig 168:  Double layer hypar.

See Section 3.2 of Reference 3. (*)

L=30;       (*) Length of side

of top layer in plan (*)

D=1.5;     (*) Distance between layers (*)

m=5;        (*) Frequency of top layer (*)

P=0.015;  (*) Pitch (*)

T=pex|rinid(m,m,2,2)|rosad(1-m,1-m)|

[-m,-m,1; 2-m,-m,1];

B=pex|rinid(m-1,m-1,2,2)|rosad(2-m,2-m)|

[1-m,1-m,0; 3-m,1-m,0];

W=rinid(m,m,2,2)|rosad(1-m,1-m)|

[-m,-m,1; 1-m,1-m,0];

Hypar=bhp(L/(2*m),L/(2*m),D,P)|

(T#B#W);

use &,vm(2),vt(2),vh(L,-L,2*L,0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 169

(*) Formian scheme for configurations

similar to Fig 169:  Compound hypar

(convex).  See Section 3.2 of

Reference 3. (*)

L=20;     (*) Length of side

of each hypar units in plan (*)

H=7;       (*) Height (*)

m=4;       (*) Semi-frequency (*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

F=bhp(L/n,L/n,1,P)|E;

Hypar=pex|rosad(L/2,L/2)|F;

use &,vm(2),vt(2),

vh(0.1*L,-4*L,4*L, 0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 170

(*) Formian scheme for configurations

similar to Fig 170:  Compound hypar

(concave).  See Section 3.2 of

Reference 3. (*)

L=20;     (*) Length of side

of each hypar units in plan (*)

H=7;      (*) Height (*)

m=4;      (*) Semi-frequency (*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

F=bhp(L/n,L/n,1,P)|E;

Hypar=pex|rosad(L/2,-L/2)|F;

use &,vm(2),vt(2),

vh(0.1*L,-3*L,3*L, 0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 171

(*) Formian scheme for configurations

similar to Fig 171:  Compound hypar.

See Section 3.2 of Reference 3. (*)

L=20;     (*) Length of side

of each hypar units in plan (*)

H=8;       (*) Height (*)

m=3;       (*) Semi-frequency (*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

F=bhp(L/n,L/n,1,P)|E;

Hypar=pex|rosad(3*L/2,3*L/2)|

rosad(L/2,L/2)|F;

use &,vm(2),vt(2),

vh(6*L,6*L,5*L,0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 172

(*) Formian scheme for configurations

similar to Fig 172:  Pellevated compound

hypar. See Section 3.2 of Reference 3

and References 4 and 5. (*)

L=20;     (*) Length of side

of each hypar units in plan (*)

H=7;       (*) Original height (*)

R=7;       (*) Pellevational rise (*)

m=4;       (*) Semi-frequency (*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

F=bhp(L/n,L/n,1,P)|E;

G=pex|rosad(L/2,L/2)|F;

Hypar=capel(3,L/2,L/2,2*L*sqrt|2,R)|G;

use &,vm(2),vt(2),

vh(0.1*L,-3*L,3*L, 0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 173

(*) Formian scheme for configurations

similar to Fig 173 :  Pellevated compound

hypar. See Section 3.2 of Reference 3

and References 4, 5 and 6. (*)

L=20;     (*) Length of side

of each hypar units in plan (*)

H=8;       (*) Original height (*)

R=35;     (*) Pellevational rise (*)

m=3;       (*) Semi-frequency (*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

F=bhp(L/n,L/n,1,P)|E;

G=pex|rosad(3*L/2,3*L/2)|

rosad(L/2,L/2)|F;

Hypar=capel(3,3*L/2,3*L/2,

4*L*sqrt|2,R)|G;

use &,vm(2),vt(2),

vh(20*L,20*L,25*L,0,0,0, 0,0,1);

clear;   draw Hypar;

برنامهء فرميينی برای تاشه های شبيه به شکل 174

(*) Formian scheme for configurations

similar to Fig 174:  Pyramid.

See Section 2.2 of Reference 2. (*)

L=20; (*) Length of each side of base (*)

H=20; (*) Height of pyramid (*)

m=7;   (*) Frequency (*)

n=3;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

E1=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

Pyramid=pex|rosad(0,0,n,360/n)|E4;

use &,vm(2),vt(2),

vh(0,-L,9*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه

به شکلهای 175 و 176

(*) Formian scheme for configurations

similar to Fig 175 (m=9, n=5) and

Fig 176 (m=6, n=12):  Pyramid.

See Section 2.2 of Reference 2. (*)

L=20; (*) Length of each side of base (*)

H=15; (*) Height of pyramid (*)

m=9;   (*) Frequency (*)

n=5;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

E1=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

Pyramid=pex|rosad(0,0,n,360/n)|E4;

use &,vm(2),vt(2),

vh(0,-L,3*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 177

(*) Formian scheme for configurations

similar to Fig 177:  Pyramid.

See Section 2.2 of Reference 2. (*)

L=20; (*) Length of each side of base (*)

H=20; (*) Height of pyramid (*)

m=8;   (*) Frequency, m must be even (*)

n=9;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

mm=ric|(m/2)-1;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E1=lux(genid(1,3,2,1,-1,1)|[0,m-2,0])|E;

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

Pyramid=pex|rosad(0,0,n,360/n)|E4;

use &,vm(2),vt(2),

vh(L,L,2.5*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 178

(*) Formian scheme for configurations

similar to Fig 178:  Pyramid.

See Section 2.2 of Reference 2. (*)

L=20; (*) Length of each side of base (*)

H=20; (*) Height of pyramid (*)

m=15; (*) Frequency (*)

n=6;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

mm=ric|(m/2)-1;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E1=lux(genid(1,5,4,2,-2,1)|[0,4,0])|E;

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

Pyramid=pex|rosad(0,0,n,360/n)|E4;

use &,vm(2),vt(2),

vh(0,2*L,5*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 179

(*) Formian scheme for configurations

similar to Fig 179:  Double layer pyramid.

See Section 2.2 of Reference 2. (*)

L=20;     (*) Length of each side of base (*)

H=12;    (*) Height of pyramid (*)

D=1.25; (*) Distance between two layers (*)

m=4;      (*) Frequency (*)

n=6;       (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

Et=genid(1,m,2,3,-1,1)|{[0,0,0; -1,3,0],

[0,0,0; 1,3,0], [-1,3,0; 1,3,0]};

Eb=genid(1,m-1,2,3,-1,1)|{[0,2,-1; -1,5,-1],

[0,2,-1; 1,5,-1], [-1,5,-1; 1,5,-1]};

Ew=genid(1,m,2,3,-1,1)|{[0,0,0; 0,2,-1],

[-1,3,0; 0,2,-1], [1,3,0; 0,2,-1]};

E1=Et#Eb#Ew;

E2=bt(L/(2*m),S/(3*m),D)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

Qi=[0,2,-1];

Ri=verat(C,0,-B)|tran(2,C-S)|

bt(L/(2*m),S/(3*m),D)|Qi;

Rj=verad(0,0,360/n)|Ri;

K1=tig|(Ri#Rj);

K2=rinax(0,0,H, -L/(2*m),

C/m,(m-1)*H/m, m)|K1;

(*) K2 represents the bottom layer

elements that link the n segments

of the pyramid (*)

Pyramid=pex|rosad(0,0,n,360/n)|(E4#K2);

use &,vm(2),vt(2),

vh(0,2*L,3*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 180

(*) Formian scheme for configurations

similar to Fig 180 :  Pellevated pyramid.

See Section 2.2 of Reference 2 and

References 4, 5 and 6. (*)

L=20; (*) Length of each side of base (*)

H=18; (*) Height of pyramid (*)

P=8;   (*) Pellevational rise (*)

m=9;   (*) Frequency (*)

n=5;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

E1=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

E5=pex|rosad(0,0,n,360/n)|E4;

Pyramid=capel(3,0,0,

L/sin|(180/n),P)|E5;

use &,vm(2),vt(2),

vh(0,-L,4.5*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 181

(*) Formian scheme for configurations

similar to Fig 181:  Pellevated Pyramid.

See Section 2.2 of Reference 2

and References 4, 5 and 6. (*)

L=20; (*) Length of each side of base (*)

H=20; (*) Height of pyramid (*)

P=6;   (*) Pellevational rise (*)

m=15; (*) Frequency (*)

n=6;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

mm=ric|(m/2)-1;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E1=lux(genid(1,5,4,2,-2,1)|[0,4,0])|E;

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

E5=pex|rosad(0,0,n,360/n)|E4;

Pyramid=capel(3,0,0,

L/sin|(180/n),P)|E5;

use &,vm(2),vt(2),

vh(0,2*L,5.5*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 182

(*) Formian scheme for configurations

similar to Fig 182:  Pellevated pyramid.

See Section 2.2 of Reference 2 and

References 4, 5 and 6. (*)

L=20; (*) Length of each side of base (*)

H=18; (*) Height of pyramid (*)

P=8;   (*) Pellevational rise (*)

m=9;   (*) Frequency (*)

n=5;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

E1=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

E5=pex|rosad(0,0,n,360/n)|E4;

Pyramid=capel(-3,0,0,

L/sin|(180/n),P)|E5;

use &,vm(2),vt(2),

vh(0,-L,2.5*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 183

(*) Formian scheme for configurations

similar to Fig 183:  Pellevated Pyramid.

See Section 2.2 of Reference 2

and References 4, 5 and 6. (*)

L=20; (*) Length of each side of base (*)

H=20; (*) Height of pyramid (*)

P=6;   (*) Pellevational rise (*)

m=15; (*) Frequency (*)

n=6;    (*) Number of sides of the base (*)

C=L/(2*tan|(180/n));

S=sqrt|(C*C+H*H);

B=atan|(H/C);

mm=ric|(m/2)-1;

E=genid(1,m,2,1,-1,1)|{[0,0,0; -1,1,0],

[0,0,0; 1,1,0], [-1,1,0; 1,1,0]};

E1=lux(genid(1,5,4,2,-2,1)|[0,4,0])|E;

E2=bb(L/(2*m),S/m)|E1;

E3=tran(2,C-S)|E2;

E4=verat(C,0,-B)|E3;

E5=pex|rosad(0,0,n,360/n)|E4;

Pyramid=capel(-3,0,0,

L/sin|(180/n),P)|E5;

use &,vm(2),vt(2),

vh(0,2*L,3.5*L, 0,0,0, 0,0,1);

clear;   draw Pyramid;

برنامهء فرميينی برای تاشه های شبيه به شکل 184

(*) Formian scheme for configurations

similar to Fig 184:  Freeform (novated

single layer grid with binate pattern and

with corners held in position (novation

mode 3 is used)). See References 9, 10

and 11. (*)

Grid=rinid(24,25,1,1)|[0,0,0; 1,0,0]#

rinid(25,24,1,1)|[0,0,0; 0,1,0];

Corner=lamid(12,12)|[0,0,0];

Base={[14,2,0], [8,18,0], [20,16,0],

[4,22,0]};

Target={[14,2,5], [8,18,0], [20,16,3],

[4,22,2]};

Form=nov(3,1,Corner#Base,

Corner#Target)|Grid;

use &,vm(2),vt(2),vh(12,-24,24,

12,12,0, 12,12,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 185

(*) Formian scheme for configurations

similar to Fig 185:  Freeform (pellevated

single layer grid with binate pattern).

See References 4, 5 and 6. (*)

Grid=rinid(40,41,1,1)|[0,0,0; 1,0,0]#

rinid(41,40,1,1)|[0,0,0; 0,1,0];

Form=capel(3,12,16,20,4)|

capel(3,18,16,24,5)|

capel(3,24,20,28,6)|Grid;

Form=rel(U(1,3)>0 || U(2,3)>0)|Form;

use &,vm(2),vt(2),vh(20,-40,120,

20,20,0, 20,20,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 186

(*) Formian scheme for configurations

similar to Fig 186:  Freeform (novated

diamatic dome with nodes along 2/3 of the

perimeter held in position (novation mode

2 is used)). See Section 2.5 of Reference 2

and References 9, 10 and 11. (*)

R=36;        (*) Radius of circumsphere (*)

A=40;        (*) Sweep angle (*)

m=8;          (*) Frequency (*)

n=6;           (*) Number of sectors (*)

E=genit(1,m,1,1,0,1)|{[1,0,0; 1,0,1],

[1,0,0; 1,1,1], [1,0,1; 1,1,1]};

F=bd(R,360/n,A/m)|E;

Dome=pex|rosad(0,0,n,360/n)|F;

Edge=bd(R,360/n,A/m)|rin(2,33,-1)|[1,m,m];

Sb=bd(R,360/n,A/m)|[1,2*m,m];

Form=nov(2,1,Edge#Sb,Edge#

tran(3,R/2)|Sb)|Dome;

use &,vm(2),vt(2),vh(R,R,3.5*R,

0,0,0, 0,0,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 187

(*) Formian scheme for configurations

similar to Fig 187:  Freeform (pellevated

diamatic dome). See Section 2.5 of

Reference 2 and References 4, 5 and 6 (*)

R=36;      (*) Radius of circumsphere (*)

A=45;      (*) Sweep angle (*)

m=9;        (*) Frequency (*)

n=7;         (*) Number of sectors (*)

E=genit(1,m,3,3,0,1)|{[1,2,4; 1,1,2],

[1,2,4; 1,1,5], [1,2,4; 1,4,5]}#

rin(3,m+1,3)|[1,1,2; 1,-1,2];

F=bd(R,360/n,A/(3*m))|E;

Dome=rosad(0,0,n,360/n)|F;

Form=verad(0,0,-45)|bapel(3,R/2,0,R/2,R/2,

0.7*R,0.2*R)|bapel(3,-R/2,0,-R/2,R/2,

0.7*R,0.2*R)|Dome;

use &,vm(2),vt(2),vh(R,R,3*R, 0,0,0, 0,0,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 188

(*) Formian scheme for configurations

similar to Fig 188:  Freeform (pellevated

compound hypar). See Section 3.2 of

Reference 3 and References 4, 5 and 6. (*)

L=20;   (*) Length of side

of each hypar units in plan (*)

H=24;   (*) Height of each unit(*)

m=8;     (*) Semi-frequency of each unit(*)

P=H/(L*L);     (*) Pitch (*)

n=2*m;

E=rinid(n,n+1,1,1)|[-m,-m,0; 1-m,-m,0]#

rinid(n+1,n,1,1)|[-m,-m,0; -m,1-m,0];

F=bhp(L/n,L/n,1,P)|E;

Hypar=pex|rosad(L/2,L/2)|F;

Form=bapel(3,0,0,0,L,L,L/2)|

bapel(3,L,0,L,L,L,L/2)|Hypar;

use &,vm(2),vt(2),

vh(0,-3*L,6*L, 0,0,0, 0,0,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 189

(*) Formian scheme for configurations

similar to Fig 189:  Freeform (pellevated

binate grid). See References 4, 5 and 6. (*)

L=40;   (*) Length of each side of grid (*)

R=8;     (*) Rise of pellevants (*)

m=24;   (*) Frequency of grid (*)

A=L/2;

Grid=bb(L/m,L/m)|

(rinid(m,m+1,1,1)|[0,0,0; 1,0,0]#

rinid(m+1,m,1,1)|[0,0,0; 0,1,0]);

Form=capel(3,0,0,L,R)|capel(3,0,L,L,R)|

capel(3,L,0,L,R)|capel(3,L,L,L,R)|

capel(1,A,A,L*sqrt|2,R)|Grid;

use &,vm(2),vt(2),

vh(A,-L,1.5*L, A,A,0, A,A,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 190

(*) Formian scheme for configurations

similar to Fig 190:  Freeform (pellevated

geodesic dome). See References

4, 5, 6, 7 and 8. (*)

RC=50; (*) Radius of circumsphere (*)

SP=30;  (*) Span of cylindrical pellevant (*)

Rise=7;  (*) Rise of pellevant (*)

m=9;      (*) Frequency of dome (*)

t=sqrt|3;

D=pol(1,5,’[1-5]’,RC,[0,0; 2*m,0])|

genid(m,m,2,t,1,-1)|rosad(1,t/3,3,120)|

[0,0; 2,0];

Geodome=dil(3,1/3)|

trac(1,[0,0,0,RC],4,-15)|D;

Form=bapel(3,0,0,0,10,SP,Rise)|Geodome;

use &,vm(2),vt(2),

vh(RC,RC,2*RC, 0,0,0, 0,0,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 191

(*) Formian scheme for configurations

similar to Fig 191:  Freeform (pellevated

ring barrel vault with binate pattern).

See References 4, 5 and 6 and Section 3.4

of Reference 3. (*)

S=60;    (*) Span of barrel vault (*)

SP=80;  (*) Span of cylindrical pellevants (*)

H=14;   (*) Rise of barrel vault (*)

RP=16; (*) Rise of pellevants (*)

m=70;   (*) Frequency in U2 direction (*)

n=10;    (*) Frequency in U4 direction (*)

R=70;   (*) Radius of annulus (*)

A=2*atan|(2*H/S);    (*) Sweep angle (*)

Q=S/(2*sin|A);          (*) Radius of

cross-section of annulus (*)

E=rinic(2,4,m,n+1,1,1)|

[1,0,1,-n/2; 1,1,1,-n/2]#

rinic(2,4,m+1,n,1,1)|

[1,0,1,-n/2; 1,0,1,1-n/2];

F=dep(4)|ba(R,360/m,Q,2*A/n)|E;

Form=bapel(3,0,0,R,0,SP,RP)|

bapel(3,0,0,0,R,SP,RP)|F;

use &,vm(2),vt(2),

vh(2*R,2*R,4*R, 0,0,0, 0,0,1);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 192

(*) Formian scheme for configurations

similar to Fig 192:  Freeform (paragenic

configuration). See Section 3.6 of

Reference 3. (*)

m=25;   (*) Frequency (*)

E=rinit(m,m+1,1,1)|[1,0,0; 1,1,0]#

rinit(m+1,m,1,1)|[1,0,0; 1,0,1];

Form=bhp(1,1,1,0.01)|verad(0,0,-45)|

tran(2,25)|verat(0,0)|bc(1,7.5,1)|

verad(0,0,45)|bc(30,80/m,60/m)|E;

use &,vm(2),vt(2),

vh(30,30,60, 0,0,30, 0,1,30);

clear;   draw Form;

برنامهء فرميينی برای تاشه های شبيه به شکل 193

(*) Formian scheme for configurations

similar to Fig 193:  Hyperboloidal tower.

See Section 3.3 of Reference 3. (*)

H=30;   (*) Height of tower (*)

H1=20; (*) Height at the neck of tower (*)

R=8;     (*) Radius at the base of tower (*)

R1=3;   (*) Radius at the neck of tower (*)

m=10;   (*) Number of sides of the base (*)

n=5;      (*) Number of rhombic units in

the vertical direction (*)

B=-2*n*H1/H;

P=(R*R-R1*R1)/(H1*H1);

E=rinit(m,n,2,2)|(lamit(0,B+1)|

[1,0,B; 1,-1,B+1]#{[1,-1,B+1; 1,1,B+1],

[1,0,B+2; 1,2,B+2]});

Tower=bh(R1,360/(2*m),H/(2*n),P)|E;

use &,vm(2),vt(2),

vh(10*R,10*R,-R, 0,0,0, 0,0,1);

clear;   draw Tower;

برنامهء فرميينی برای تاشه های شبيه به شکل 194

(*) Formian scheme for configurations

similar to Fig 194:  Hyperboloidal tower.

See Section 3.3 of Reference 3. (*)

H=60;   (*) Height of tower (*)

H1=50; (*) Height at the neck of tower (*)

R=8;     (*) Radius at the base of tower (*)

R1=3;   (*) Radius at the neck of tower (*)

m=6;     (*) Number of sides of the base (*)

n=8;      (*) Number of rhombic units in

the vertical direction (*)

B=-2*n*H1/H;

P=(R*R-R1*R1)/(H1*H1);

E=rinit(m,n,2,2)|(lamit(0,B+1)|

[1,0,B; 1,-1,B+1]#{[1,-1,B+1; 1,1,B+1],

[1,0,B+2; 1,2,B+2]});

Tower=bh(R1,360/(2*m),H/(2*n),P)|E;

use &,vm(2),vt(2),

vh(10*R,10*R,-4*R, 0,0,0, 0,0,1);

clear;   draw Tower;

برنامهء فرميينی برای تاشه های شبيه به شکل 195

(*) Formian scheme for configurations

similar to Fig 195:  Conical tower.

See Section 3.3 of Reference 3. (*)

H=20;   (*) Height of tower (*)

H1=80; (*) Height at the neck of tower (*)

R=8;     (*) Radius at the base of tower (*)

R1=0;   (*) Radius at the neck of tower (*)

m=10;   (*) Number of sides of the base (*)

n=4;      (*) Number of rhombic units in

the vertical direction (*)

B=-2*n*H1/H;

P=(R*R-R1*R1)/(H1*H1);

E=rinit(m,n,2,2)|(lamit(0,B+1)|

[1,0,B; 1,-1,B+1]#{[1,-1,B+1; 1,1,B+1],

[1,0,B+2; 1,2,B+2]});

Tower=bh(R1,360/(2*m),H/(2*n),P)|E;

use &,vm(2),vt(2),

vh(10*R,10*R,-6*R, 0,0,0, 0,0,1);

clear;   draw Tower;

برنامهء فرميينی برای تاشه های شبيه به شکل 196

(*) Formian scheme for configurations

similar to Fig 196:  Conical tower.

See Section 3.3 of Reference 3. (*)

H=22;   (*) Height of tower (*)

H1=30; (*) Height at the neck of tower (*)

R=15;   (*) Radius at the base of tower (*)

R1=0;   (*) Radius at the neck of tower (*)

m=10;  (*) Number of sides of the base (*)

n=4;     (*) Number of rhombic units in

the vertical direction (*)

B=-2*n*H1/H;

P=(R*R-R1*R1)/(H1*H1);

E=rinit(m,n,2,2)|(lamit(0,B+1)|

[1,0,B; 1,-1,B+1]#{[1,-1,B+1; 1,1,B+1],

[1,0,B+2; 1,2,B+2]});

Tower=bh(R1,360/(2*m),H/(2*n),P)|E;

use &,vm(2),vt(2),

vh(10*R,10*R,-2*R, 0,0,0, 0,0,1);

clear;   draw Tower;

برنامهء فرميينی برای تاشه های شبيه به شکل 197

(*) Formian scheme for configurations

similar to Fig 197:  Cylindrical tower.

See Section 3.3 of Reference 3. (*)

H=20;   (*) Height of tower (*)

H1=20; (*) Height at the neck of tower (*)

R=8;     (*) Radius at the base of tower (*)

R1=8;   (*) Radius at the neck of tower (*)

m=12;  (*) Number of sides of the base (*)

n=4;     (*) Number of rhombic units in

the vertical direction (*)

B=-2*n*H1/H;

P=(R*R-R1*R1)/(H1*H1);

E=rinit(m,n,2,2)|(lamit(0,B+1)|

[1,0,B; 1,-1,B+1]#{[1,-1,B+1; 1,1,B+1],

[1,0,B+2; 1,2,B+2]});

Tower=bh(R1,360/(2*m),H/(2*n),P)|E;

use &,vm(2),vt(2),

vh(10*R,10*R,-R, 0,0,0, 0,0,1);

clear;   draw Tower;

برنامهء فرميينی برای تاشه های شبيه به شکل 198

(*) Formian scheme for configurations

similar to Fig 198:  Lazy-tongs.

See Section 2.4 of Reference 2. (*)

T=90;  (*) Control angle (*)

D=1.2;  (*) Dimensions

of a duplet for T=90 (*)

m=5;    (*) Frequency of duplets (*)

H=sqrt|2*D*sin|(T/2);

V=sqrt|2*D*cos|(T/2);

Fold=rin(1,m,H)|{[0,0,0; H,0,V],

[0,0,V; H,0,0]};

use &,vn(250,150),vs(35),

vh(0, m*H,0, 0,0,0, 0,0,1);

clear;   draw Fold;

برنامهء فرميينی برای تاشه های شبيه به شکل 199

(*) Formian scheme for configurations

similar to Fig 199:  Foldable double layer

grid. See Section 2.4 of Reference 2. (*)

T=120; (*) Control angle (*)

D=0.8; (*) Dimensions

of a duplet for T=90 (*)

m=6;    (*) Frequency in the x-direction (*)

n=5;     (*) Frequency in the y-direction (*)

H=sqrt|2*D*sin|(T/2);

V=sqrt|2*D*cos|(T/2);

Fold=rinid(m,n+1,H,H)|{[0,0,0; H,0,V],

[0,0,V; H,0,0]}#rinid(m+1,n,H,H)|

{[0,0,0; 0,H,V], [0,0,V; 0,H,0]};

use &,vn(150,250),vs(35),

vh(m*H, n*H,15*H, 0,0,0, 0,0,1);

clear;   draw Fold;

برنامهء فرميينی برای تاشه های شبيه به شکل 200

(*) Formian scheme for configurations

similar to Fig 200:  Foldable double layer

barrel vault. See Section 2.4 of

Reference 2. (*)

T=140;  (*) Control angle (*)

L1=0.8; (*) Length of

upper part of uniplet (*)

L2=0.7; (*) Length of

lower part of uniplet (*)

m=7;     (*) Circumferential frequency (*)

n=5;      (*) Longitudinal frequency (*)

L =L1+L2;   T1=180-T;

D=sqrt|(L1*L1+L2*L2-2*L1*L2*cos|T1);

D1=sqrt|(L*L-D*D);

A=asin|(L2*sin|T1/D);

B=180+A-T;

R=D*sin|B/(sin|B-sin|A);

C=asin|(L*sin|B/R);

E=rinit(m,n+1,C,D1)|{[R-D,0,0; R,C,0],

[R,0,0; R-D,C,0]}#rinit(m+1,n,C,D1)|

{[R-D,0,0; R,0,D1], [R,0,0; R-D,0,D1]};

F=bc(1,1,1)|E;   P=m*C/2;

Fold=verad(0,0,90-P)|F;

use &,vn(150,100),vs(30),

vh(5,8*R,-12*R, 0,0,R, 0,1,R);

clear;   draw Fold;

References

- Nooshin H and Disney P L, Formex Configuration Processing I, International Journal of Space Structures, Vol 15, 2000

- Nooshin H and Disney P L, Formex Configuration Processing II, International Journal of Space Structures, Vol 16, 2001

- Nooshin H and Disney P L, Formex Configuration Processing III, International Journal of Space Structures, Vol 17, 2002

- Nooshin H, A Technique for Surface Generation, IASS Symposium, Stuttgart, Germany, October 1996

- Hofmann I S, The Concept of Pellevation for Shaping of Structural Forms, PhD Thesis, University of Surrey, UK, 1999

- Hofmann I S, The Concept of Pellevation for Shaping of Structural Forms, Fifth International Conference on Space Structures, University of Surrey, Published by Thomas Telford, 2002

- Nooshin H, Disney P L and Champion O C, Computer-Aided Processing of Polyhedric Configurations, Chapter 12 in Beyond the Cube, Edited by J F Gabriel, John Wiley, 1997

- Champion O C, Polyhedric Configurations, PhD Thesis, University of Surrey, UK, 1997

- Nooshin H, Albermani F G A and Disney P L, Novational Transformations, IASS Colloquim on Structural Morphology, Nottingham, UK, August 1997

- Nooshin H and Moghimi M, Formex Formulation of Freeform Structural Surfaces, Second National Conference on Space Structures, Teheran, Iran, May 2007

- Moghimi M, Formex Configuration Processing of Compound and Freeform Structures, PhD Thesis, University of Surrey, UK, 2006

- Nooshin H, Tomatsuri H and Fujimoto M, Scallop Domes, IASS Symposium, Singapore, November 1997